| TEXTILES PRODUCTS |  |  |  |  |
| --- | --- | --- | --- | --- |
| Sl. No. | Export Item | Quantity | ## Import Item | Qty. Allowed |

| J1 | 100% Carbonised Polyester, bleached, dyed, printed fabrics/ Made ups, Sarees, Dhotis, Udhanies with or without embroidery, with or without emboss | 1 Kg | 1 |  | Polyester Staple Fibre | 1.143 kgs. per kg of polyester content in the export product. |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 2 |  | Disperse dyes of 100% strength (Not to be allowed if the product is not dyed or printed) | 2% weight of polyester content in the export. |
|  |  |  | 3 |  | Optical Whitening Agent (Only for white fabrics or printed fabric having white ground) | 0.005 Kg. |
|  |  |  | 4 |  | Polyvinyl Alcohol | 0.01 Kg. |
|  |  |  | 5 |  | Silicon emulsion | 0.02 Kg. |
|  |  |  | 6 |  | Hydrogen Peroxide | 0.02 Kg. |
|  |  |  | 7 |  | Sodium Sulfoxylate Formaldehyde (Rangolite, Sulfolite, Jaykelite etc.) (to be allowed only in case of printed material) | 0.01 Kg. |
|  |  |  | 8 |  | Viscose Staple Fibre | 1.09 Kg/Kg of Viscose material content in the uncarbonised fabric of the export product. |
| J2 | 100% Dyed Acrylic Fabric | 1 kg | 1 |  | Acrylic Fibre | 1.06 kg/kg content in export. |
|  |  |  | 2 |  | Cationic Dyes of 100% strength | 2% of weight of Acrylic content in the export product. |
| J3 | 100% Polyester grey sewing thread | 1.0 Kg | 1 |  | 100% Polyester grey yarn | 1.01 Kg |
| J4 | 100% Polyester Sewing Thread Dyed | 1 kg | 1 |  | Polyester Fibre upto 5 denier | 1.025 kg/kg content in export product. |
|  |  |  | 2 |  | Disperse Dyes (100% strength) | 0.02 kg |
|  |  |  | 3 |  | Lubricating agent | 0.05 kg/kg (50% concentration) |
|  |  |  | 4 |  | Cone centres | net to net basis |
| J5 | Acrylic Blended knitwear containing Acrylic Fibre and Cotton Fibre | 1 kg | 1 |  | Acrylic Fibre | 1.30 kg/kg content in the export product. |
|  |  |  | 2 |  | Cotton Fibre | 1.15 kg/kg content in the export product. |
| J6 | Acrylic Dyed Yarn (under cotton system) | 1.0 Kg | 1 |  | Acrylic Fibre | 1.06 Kg./Kg. content in export. |
|  |  |  | 2 |  | Cationic Dyes of 100% strength | 0.02 Kg. |
|  |  |  | 3 |  | Optical Whitener | 0.005 kg. |
|  |  |  | 4 |  | Antistatic Oil | 0.005 kg. |
| J7 | Acrylic Dyed Yarn (under worsted system) | 1.0 Kg | 1 |  | Acrylic Fibre | 1.11 Kg./Kg.content in export. |
|  |  |  | 2 |  | Cationic Dyes of 100% strength | 0.02 Kg. |
| J8 | Acrylic Shawls/ Mufflers/ Scarves | 1 kg | 1 |  | Acrylic fibres | 1.30 kg |
|  |  |  | 2 |  | Acrylic Dyes | 1.5% on the weight of Acrylic content in export. |
| J9 | Acrylic Yarn | 1 Kg | 1 |  | Acrylic tow | 1.05 Kg/kg content in export |
| J10 | Acrylic/ Cotton Dyed Yarn | 1 Kg | 1 |  | Acrylic Fibre | 1.06 Kg/Kg content in the export product. |
|  |  |  | 2 |  | Acrylic Dyes of 100% strength | 2% of weight of Acrylic content in the export product. |
|  |  |  | 3 |  | Reactive/ Vat Dyes of 100% strength | 1.5% of weight of Cotton Yarn content in the export product |
| J11 | Acrylic/cotton Grey yarn | 1 Kg | 1 |  | Acrylic Fibre | 1.05 Kg./Kg. content in export. |
| J12 | Acrylic/Polyester Blended Dyed spun yarn. | 1 Kg | 1 |  | Acrylic Fibre | 1.06 Kg./Kg. content in export product. |
|  |  |  | 2 |  | Polyester Fibre | 1.06 Kg./Kg. content in export product. |
|  |  |  | 3 |  | Acrylic Dyes of 100% strength | 2% of wt. of Acrylic content in export |
|  |  |  | 4 |  | Disperse Dyes (100% Strength) | 0.02 Kg. |
| J13 | Acrylic/Polyester Blended grey yarn | 1 Kg | 1 |  | Acrylic Fibre | 1.05 Kg./Kg. content in export |
|  |  |  | 2 |  | Polyester Fibre | 1.05 Kg./Kg. content in export |
| J14 | Art Silk Dyed & Bleached Embroidery Thread (made of 100% Viscose artificial Filaments Yarn) | 1 Kg | 1 |  | Viscose Filament Yarn | 1.02 Kg. per Kg. content in export. |
| J15 | Artificial Fur Fabrics | 1.0 Kg | 1 |  | Acrylic Fibre upto 10 denier | 1.22 Kg./Kg.content in export. |
|  |  |  | 2 |  | Partially Oriented Yarn | 1.07 Kg./Kg. content in export. |
|  |  |  | 3 |  | Disperse dyes of 100% strength | 2% of weight of Polyester content in export. |
|  |  |  | 4 |  | Conning Oil | 2% of weight of Polyester content in export. |
|  |  |  | 5 |  | Cationic dyes of 100% strength | 2 % of weight of Acrylic content in export. |
|  |  |  | 6 |  | Polyacrylic Co-polymer Emulsion | 0.02 Kg. |
| J16 | Artificial Fur Lining | 1 Kg | 1 |  | Acrylic Fibre | 1.22 Kg./Kg. content in export. |
|  |  |  | 2 |  | Polyester Texturised Yarn | 1.06 Kg./Kg. content in export. |
|  |  |  | 3 |  | Cationic dyes of 100% strength. | 2% of Acrylic content in export. |
|  |  |  | 4 |  | Disperse dyes of 100% strength | 2% of polyester content in export. |
|  |  |  | 5 |  | Polyacrylic co-polymer emulsion | 0.05 Kg. |
| J17 | Bleached Knitted Fabrics containing Polyester/Cotton | 1 kg | 1 |  | Knitted Fabric of relevant Polyester/Cotton Blend (Grey) | 1.02 kgs |
|  |  |  | 2 |  | Hydrogen Peroxide | 2% of weight of Cotton content in export product. |
|  |  |  | 3 |  | Optical Whitening Agent | 0.5% of weight of Fabric. |
| J18 | Bleached or Processed Polyester/ Cotton Blended Fabrics or Made-ups | 1 kg | 1 |  | Polyester Staple Fibre | 1.18 kg/kg content in export. |
|  |  |  | 2 |  | Polyvinyl Alcohol | 0.01 kg |
| J19 | Bleached, Dyed/Printed Man-made Fabrics, Sarees Embroidered with Metallic Yarn & Glass Beads | 1 kg | 1 |  | Polyester Staple Fibre(below 6 denier) | 1.115 kg/kg content in the export product. |
|  |  |  | 2 |  | Viscose Fibre (below 6 denier) | 1.115 kg/kg content in export product. |
|  |  |  | 3 |  | Metallic Yarn | 1.05 kg/kg of Metallic Yarn in export product. |
|  |  |  | 4 |  | Glass Beads | Net to Net. |
|  |  |  | 5 |  | Disperse Dyes of 100% Strength | 2% of weight of Polyester content in export product. |
|  |  |  | 6 |  | Reactive/ Azo/ Vat Dyes (100% strength) | 1.5% of weight of viscose in export product |
|  |  |  | 7 |  | Polyvinyl alcohol | 1% of the weight of Fabric(excluding beads) |
|  |  |  | 8 |  | Optical Whitening Agent | 0.5% of weight of fabric in case of bleached white only. |
|  |  |  | 9 |  | Hydrogen Peroxide | 2% of weight of fabric in case of Bleached white only. |
|  |  |  | 10 |  | Silicon Emulsion | 2% on the weight of Fabric(excluding beads) |
| J20 | Deleted |  |  |  |  |  |
| J21 | Blended Polyester/Cotton Grey Fabrics or Made-ups | 1.0 Kg | 1 |  | Polyester fibre | 1.16 Kg/Kg content in the export product. |
|  |  |  | 2 |  | Polyvinyl Alcohol OR Hydroxyl Ether of Potato Starch OR Modified Polysaccharide | 0.01 kg 0.058 kg 0.016 kg |
|  |  |  | 3 |  | Raw Cotton | 1.26 kg/kg content in the export product |
| J22 | Blended Yarn (containing Flax and Viscose) | 1 kg | 1 |  | Flax Fibre | 1.66 kg./kg. content in export. |
|  |  |  | 2 |  | Viscose staple Fibre | 1.05 kg./kg. content in export. |
| J23 | Both side Fleece Fabric Dyed Printed containing Spun Polyester Yarn and Polyester Filament Yarn | 1 kg. | 1 |  | 100% Spun Polyester yarn | 1.135 kg./kg of polyester yarn content in export product. |
|  |  |  | 2 |  | POY/PFY | 1.10 Kg/Kg of POY/PFY content in the export product. |
|  |  |  | 3 |  | Disperse Dyes of 100% strength | 0.02 kg. (0.04 kg for dyed and printed material). |
|  |  |  | 4 |  | Softening Agent | - kg. |
|  |  |  | 5 |  | Fleecing Lubricant Agent | 0.01 kg. |
|  |  |  | 6 |  | Optical Whitener (only for printed fabric having white ground) | 0.005 kg. |
| J24 | Both side Fleece Knitted Fabric Dyed/ Printed containing Spun Polyester Yarn and Polyester Filament Yarn | 1 Kg. | 1 |  | 100% Polyester knitted Grey Fabric made out of polyester spun yarn and polyester filament yarn. | 1.05 kg/kg content in export product. |
|  |  |  | 2 |  | Disperse dyes of 100% strength | 0.02 Kg/kg of Polyester content in the export product. |
|  |  |  | 3 |  | Softening agent | 0.01 Kg |
|  |  |  | 4 |  | Fleecing Lubricant. | 0.01 Kg |
|  |  |  | 5 |  | Optical Whitener (only for printed fabric having white ground) | 0.005 Kg |
|  |  |  | 6 |  | Rangolite (for printed material only) | 0.01 Kg. |
| J25 | Coated Cotton Fabric PVC Leather Cloth Metallised(Thickness 0.35mm +/- 0.1mm coating weight 288 gms +/- 5% | 1 Sq.Mtr. | 1 |  | PVC Resin | 169.47 gms |
|  |  |  | 2 |  | Plasticizer(DOP/DOA) | 117.80 gms |
|  |  |  | 3 |  | Polyurethene Resin | 21.00.gms |
|  |  |  | 4 |  | Cross Linking Agents/Accelerators | 7.64 gms |
|  |  |  | 5 |  | Stamping Foil | 1.05 Sq.Mtr. |
| J26 | Cotton & Acrylic blended yarn. | 1 kg. | 1 |  | Raw Cotton | 1.10 kg/ kg of cotton content in the end product |
|  |  |  | 2 |  | Acrylic fibre | 1.05 kg/ kg of Acrylic content in the end product |
| J27 | Combed Cotton Core Spun Yarn/ Sewing Thread | 1 kg | 1 |  | Raw cotton | 1.22 kg/kg of material content in the export product |
|  |  |  | 2 |  | Relevant Core yarn | 1.01 kg/kg of material content in the export product |
| J28 | Dyed/Dyed & Printed Fabrics/Sarees/Made-ups made from 100% Polyester Filament yarn (By weight reduction process) | 1 kg | 1 |  | Polyester filament Yarn OR | 1.180 kgs per kg content in the export product. |
|  |  |  |  |  | Partially Oriented Polyester Filament Yarn | 1.180 kgs per kg content in the export product. |
|  |  |  | 2 |  | Polyvinyl alcohol | 0.010 kg. |
|  |  |  | 3 |  | Sodium Sulphoxylate formaldehyde OR | 0.010 kg. |
|  |  |  |  |  | Zinc Sulphoxylate formaldehyde (e.g. Rangolite DS/DP/C, Jaykaylite Saffolite etc.) | 0.010 kg. |
|  |  |  | 4 |  | Silicon Compound | 0.020 kg. |
|  |  |  | 5 |  | Optical Whitening Agent | 0.005 kg. |
|  |  |  | 6 |  | Ionet Rap (levelling agent for disperse dyes) | 0.010 kg. |
|  |  |  | 7 |  | Disperse dyes of 100% Strength | 2% of wt of Polyester content (only for dyed goods) (4% of weight of polyester for dyed and printed material) |
|  |  |  | 8 |  | Lubricating Agent | 0.002 kg. |
|  |  |  | 9 |  | Antistatic Agent | 0.001 kg. |
|  |  |  | 10 |  | Printing Gum | 0.020 kg. |
| Note: 1. The exporter at the time of exports will supply a sample of grey and processed fabric to the Customs Authority supported by a Certificate issued from the testing laboratories of the Textile Committee/Man made Textiles Research Association(MANTRA)/ Silk & Artsilk Mills Research Association(SASMIRA),Bombay/ Bombay Textiles Research Association(BTRA),Bombay/ Southern India Textiles Research Association (SITRA), Coimbatore/Ahmedabad Textile covers on(SITRA), Coimbatore/ Ahmedabad Textile Industries Research Association (ATIRA), Ahmedabad or any other institution recognised by the Government of India for this purpose. 2. Item 6 & 10 are to be allowed only for printed material. 3. Item 5 is to be allowed only for printed material having white ground. |  |  |  |  |  |  |
| J29 | Dyed and Multiplied Viscose Rayon Filament Embroidery Thread | 1 kg | 1 |  | Viscose Rayon Filament Yarn | 1.01 kg/kg content in the export product. |
|  |  |  | 2 |  | Sulphur/Fibre Reactive/VAT Dyes(100% Strength) | 0.015 kg |
| J30 | Dyed/Dyed and Printed Fabrics/ Sarees/ made ups/ Hosiery/ Odhanies made from 100% Polyester filament Yarn/ texturised yarn. | 1 kg | 1 |  | Polyester filament Yarn OR | 1.10 kg./ kg content in export. |
|  |  |  |  |  | Partially Oriented Polyester Filament Yarn | 1.10 kg./ kg content in export. |
|  |  |  | 2 |  | Polyvinyl Alcohol | 0.010 kg. |
|  |  |  | 3 |  | Sodium Sulphoxylate formaldehyde OR | 0.01 kg.. |
|  |  |  |  |  | Zinc Sulphoxylate Formaldehyde (e.g. Rangolite DS/DP/C, Jaykeylite, Saffolite etc.) | 0.01 kg. |
|  |  |  | 4 |  | Sillicon compound | 0.020 kg. |
|  |  |  | 5 |  | Optical whitening agent | 0.005 kg. |
|  |  |  | 6 |  | Ionet Rap (levelling agent for disperse dyes) | 0.010 kg. |
|  |  |  | 7 |  | Disperse dyes of 100% strength | 2% of wt. of polyester content (only for dyed goods) (4% of weight of polyester for dyed and printed material) |
|  |  |  | 8 |  | Lubricating agent | 0.002 kg. |
|  |  |  | 9 |  | Antistatic agent | 0.001 kg. |
|  |  |  | 10 |  | Printing Gum (only for printed material) | 0.020 kg. |
| Note.1 Item 5 will not be allowed in case of Hosiery Products. |  |  |  |  |  |  |
| J31 | Bleached/Dyed and Printed Polyester Cotton Blended farbic | 1 Kg | 1 |  | Polyester fibre OR Polyester Cotton Blended Grey Fabric | 1.211 Kg./Kg. content in export. 1.03 kg/kg content in the export product. |
|  |  |  | 2 |  | Polyvinyl Alcohol | 0.02 kg |
|  |  |  | 3 |  | Disperse Dyes of 100% strength | 2% on the weight of polyester content in export product |
|  |  |  | 4 |  | Vat Dyes of 100% strength | 1.5% on the weight of cotton content in export product |
|  |  |  | 5 |  | Optical Whitener | 0.5% on the weight of export product in the case of white discharge printed fabric |
|  |  |  | 6 |  | Bronze powder | 1% on the weight of export product if the fabric contain gold print |
|  |  |  | 7 |  | Silicon Emulsion | 0.02 kg |
|  |  |  | 8 |  | Rangolite | 0.01 kg (In case of discharge printed fabric only) |
| Note: 1. Fabric made out of Polyester Cotton Dyed Blended Yarn shall also get covered with the ambit of above mentioned resultant product. If the import material is Polyester/Cotton Blended Grey Fabric, the item PVA shall not be permitted.2. If the export item is not dyed one, the import of items No.3&4 above, i.e. dyes shall not be allowed. |  |  |  |  |  |  |
| J32 | Dyed and/or Printed Synthetic Blanket made of Acrylic Fibre and/or acrylic yarn and polyester filament yarn | 1 kg | 1 |  | Acrylic fibre OR Acrylic Yarn | 1.24 kg/kg content in export product. 1.15 kg/kg content in the export product. |
|  |  |  | 2 |  | Polyester Filament Yarn | 1.05 kg/kg. content in the export product. |
|  |  |  | 3 |  | Lylex cloth | Net to net. |
|  |  |  | 4 |  | Cationic Dyes (Acrylic Dyes 100% strength) | 2% of Acrylic content in export |
| J33 | Dyed draw Texturised Synthetic Filament Yarn/Dyed Texturised Synthetic Filament Yarn | 1 Kg | 1 |  | Partially Oriented Synthetic Yarn/ Fully Drawn Yarn (Relevant Type/Quality of yarn to be allowed) | 1.02 Kg/kg content in export. |
|  |  |  | 2 |  | Acid/Pre Metallised Dyes (100% strength) | 0.02 kg |
| J34 | Finished Flax Blended Fabrics (Grey or Dyed) containing Polyester/ Viscose/ Cotton/ Flax | 1 kg | 1 |  | Flax Fibre OR | 1.843 kg/kg of Flax Fibre content in the export product. |
|  |  |  |  |  | Flax Yarn OR | 1.07 kg/kg of Flax content in the export product. |
|  |  |  |  |  | Flax Blended Yarn OR | 1.11 kg/kg content in the export product. |
|  |  |  |  |  | Flax Blended Fabrics (Grey) | 1.05 kg/kg content in the export product. |
|  |  |  | 2 |  | Polyester Fibre OR | 1.115 kg/kg of Polyester content in the export product. |
|  |  |  |  |  | Polyester Yarn | 1.065 kg/kg of Polyester content in the export product. |
|  |  |  | 3 |  | Viscose Fibre OR | 1.115kg/kg of Viscose content in the export product |
|  |  |  |  |  | Viscose Yarn | 1.065 kg/kg of Viscose content in the export product. |
|  |  |  | 4 |  | Disperse Dyes of 100% Strength | 2% of weight of Polyester content in the export product. |
|  |  |  | 5 |  | Vat Dyes of 100% Strength | 0.015 kg/kg of Flax, Viscose and Cotton content in the export product. |
| Note: Items 4 and 5 shall be allowed only if export product is Dyed one |  |  |  |  |  |  |
| J35 | Dyed Knitted Fabrics containing Polyester Viscose Yarn | 1 kg | 1 |  | Polyester Viscose Yarn | 1.08 kg/kg content in the export product. |
|  |  |  | 2 |  | Disperse Dyes (100% Strength) | 2% of weight of Polyester content in the export product. |
|  |  |  | 3 |  | Vat Dyes (100% Strength) | 1.5% of weight of Viscose content in export product. |
|  |  |  | 4 |  | Optical Whitening Agent | 0.005 kg |
| J36 | Dyed Polyester/ Wool blended Fabrics | 1 kg | 1 |  | Polyester Fibre (3 Denier) OR Polyester Tow | 1.211 kg/kg content in export product. 1.170 kg/kg content in export product. |
|  |  |  | 2 |  | Scoured Wool | 1.308 kg/kg content in export product. |
|  |  |  | 3 |  | Tinting Agent | 0.005 kg |
|  |  |  | 4 |  | Disperse Dyes (100% strength) | 2% of weight of Polyester content in export item. |
|  |  |  | 5 |  | Acid Dyes/ Metal Complex Dyes (100% strength) | 2% of weight of Wool content in export item. |
| Note: In case, the export product is Teflon Treated, an additional item Oleophobol-S/ Hydrophobol-X may be allowed @0.04 kg/kg of the export product. |  |  |  |  |  |  |
| J37 | Dyed/ Printed Polyester Viscose Blended Fabrics | 1 Kg | 1 |  | Polyester Filament Yarn/POY | 1.10 Kg per kg of filament content in the export product |
|  |  |  | 2 |  | Viscose filament yarn | 1.10 kg/kg of filament content in export product |
|  |  |  | 3 |  | Polyester Viscose blended yarn | 1.065 kg per kg of blended yarn content in the export product |
|  |  |  | 4 |  | Disperse Dyes of 100% strength | 2% of the weight of polyester content in the export product. |
|  |  |  | 5 |  | Optical Whitening Agent | 0.005 Kg |
|  |  |  | 6 |  | Silicon Emulsion | 0.02 Kg. |
|  |  |  | 7 |  | Vat Dyes | 1.5% of weight of Viscose content in the export product |
|  |  |  | 8 |  | Bronze powder | 0.01 kg (in case of Golden prints only) |
| J38 | Fabric manufactured out of corespun yarn(Synthetic Filament core & cotton sheath) | 1 kg. | 1 |  | Relevant Synthetic Filament yarn | 1.11 kg./kg.content in export product. |
| J39 | Fabrics made out of polyester filament yarn/ nylon filament yarn/polyester metallised lurex yarn and viscose filament yarn/dyed acrylic Spun yarn | 1 Kg | 1 |  | Polyester Filament Yarn | 1.10 Kg./Kg. content in export. |
|  |  |  | 2 |  | Nylon Filament yarn | 1.10 Kg./Kg. content in export. |
|  |  |  | 3 |  | Polyester metallised yarn | 1.05 Kg./Kg. content in export. |
|  |  |  | 4 |  | Viscose Filament yarn | 1.10 Kg./Kg. content in export. |
|  |  |  | 5 |  | Disperse dyes of 100% strength | 2% of weight of polyester content in export. |
|  |  |  | 6 |  | Acid Dyes of 100% strength | 2% of weight of nylon content. |
|  |  |  | 7 |  | Dyed acrylic Spun yarn | 1.05 kg/kg content in export. |
|  |  |  | 8 |  | Vat Dyes of 100% Strength | 1.5% of weight of Viscose content in export |
|  |  |  | 9 |  | Poly Vinyl Alcohol | 1% of weight of export |
| J40 | Fabrics/Sarees/ Made ups made from Manmade filament Yarn and metallised yarn | 1 kg | 1 |  | Man made filament yarn | 1.10 kgs/ kg of Man made filament yarn content in the export product |
|  |  |  | 2 |  | Metallised Yarn | 1.05 kgs/ kg of Metallised yarn in the export product |
|  |  |  | 3 |  | Relevant Dyes | 1.5% of man made filament yarn in the export product. |
| J41 | Fancy Fleece/Fleece Fabric containing Polyester Filament/ Spun Yarn and other blended yarn of different blends | 1 kg | 1 |  | Polyester Filament Yarn | 1.10 kg/kg content in export product. |
|  |  |  | 2 |  | Polyester Spun/ Blended Yarn | 1.12 kg/kg content in export product. |
|  |  |  | 3 |  | Disperse Dyes of 100% strength (To be allowed only if the export product is dyed or printed) | 0.02 kg/kg of Polyester content in export product |
|  |  |  | 4 |  | Vat/Sulphur/ Reactive Dyes of 100% Strength (To be allowed only if the export product is dyed or printed) | 0.015 kg/kg of Cellulosic Materials content in export product. |
|  |  |  | 5 |  | Softening Agent | 0.01 kg |
|  |  |  | 6 |  | Fleecing Agent | 0.005 kg |
|  |  |  | 7 |  | Optical Whitener (for white fabric only) | 0.005 kg |
| J42 | Fancy/Metallic yarn with or without Viscose/Nylon yarn. | 1 kg | 1 |  | Viscose Filament Yarn | 1.01 kg/kg content in the export product. |
|  |  |  | 2 |  | Metallic Yarn | 1.01 kg/kg content in the export product. |
|  |  |  | 3 |  | Nylon Yarn | 1.10 kg/kg content in the export product |
| J43 | Flax Cotton Blended Yarn | 1 kg | 1 |  | Flax Fibre | 1.3 kg/kg content in export. |
| J44 | Flax Fabrics dyed | 1 Kg | 1 |  | Flax Yarn OR | 1.07 Kgs. |
|  |  |  |  |  | Flax Fibre | 1.78 kg |
|  |  |  | 2 |  | Textile Dyes/Vat/Reactive of 100% strength | 0.015 Kg. |
| J45 | Fur Fabric (Knitted) | 1 kg. | 1 |  | Acrylic Fibre | 1.15 kg/ kg content in export |
| J46 | Grey Knitted Fabrics containing Elastic Yarn | 1 kg | 1 |  | Elastic Yarn | 1.03 kg/kg of material content in the export product. |
| J47 | Grey/Draw Texturised Filament Yarn | 1 kg | 1 |  | Partially Oriented Synthetic Yarn/Fully Drawn Yarn(Relevant Type/quality of yarn to be allowed) | 1.01 kg/kg content in export |
| J48 | Grey/draw Texturised Filament Yarn/ Texturised Filament Yarn | 1 Kg | 1 |  | Partially Oriented Polyester Yarn or Polyester Filament Yarn | 1.01 Kg/kg content in export. |
| J49 | Grey/dyed and/ or Printed Fabrics/Made-ups (including quilted made ups) made from relevant man made fibres or yarns | 1 kg | 1 |  | Relevant Man Made Fibre OR Relevant Man Made Yarn | 1.115 kg/kg content of the import item in the export product. 1.05 kg/kg content of the import item in the export product |
|  |  |  | 2 |  | Metallic Yarn | 1.05 kg/kg of Metallic Yarn in the export product. |
|  |  |  | 3 |  | Glass Beads (Relevant) | Net content + 1% wastage. |
|  |  |  | 4 |  | Disperse Dyes of 100% Strength (Not to be allowed if the export product is not dyed or printed) | 2% of wt. of Polyester content in the export product. |
|  |  |  | 5 |  | Vat Dyes of 100% Strength (Not to be allowed if the export product is not dyed or printed) | 1.5% of wt. of Viscose in the export product. |
|  |  |  | 6 |  | Polyvinyl Alcohol | 1% of Polyester content in the export product. |
|  |  |  | 7 |  | Hydrogen Peroxide | 2% of wt. of fabric in case of printed material having white background. |
|  |  |  | 8 |  | Silicon Emulsion | 2% on the weight of export product and (4% in case of dyed and printed material) |
|  |  |  | 9 |  | Raw Polyfill | 1.02 kg/kg content in the export product |
| J50 | Grey/Dyed Spun Yarn made out of Man made Synthetic Staple Fibres (Polyester, Viscose etc) and the blends thereof | 1 Kg | 1 |  | Relevant Man made Staple Fibre | 1.05 Kg/Kg of relevant fibre content in export. |
|  |  |  | 2 |  | Polyster /Viscose Tow | 1.03 Kg/Kg content in export. |
|  |  |  | 3 |  | Disperse Dyes | 2% of weight of Polyester content in export. |
|  |  |  | 4 |  | Reactive/Vat/Sulphur Dyes of 100% strength OR | 1.5% of wt. of viscose content in Export |
|  |  |  | 1 |  | Manmade/Synthetic Staple Fibre/Top | 1.03 kg/kg content in export. |
|  |  |  | 2 |  | Disperse Dyes of 100% strength (for dyed goods only) | 2% of Polyester content in the export product. |
|  |  |  | 3 |  | Vat/Sulphur/Reactive dyes of 100% strength (for dyed goods only) | 1.5% of Cellulosic content in the export product. |
|  |  |  | 4 |  | HSD OR | 1.00 litre |
|  |  |  |  |  | Furnace Oil | 0.56 litre |
| Note: In case import item is polyester staple fibre dyed and viscose staple fibre dyed then import of disperse dyes shall not be permissible Note 2. Furnace Oil is allowed only under actual user condition. It is applicable to units having facilities for captive power generation using Furnace Oil only. |  |  |  |  |  |  |
| J51 | Grey/Dyed/Printed Fabrics with Embroidery | 1 Sq.mtr. | 1 |  | Relevant Fabric | 1.01 Sq.Mtr. |
| J52 | Knitted Hosiery Fabric Dyed containing polyester/Cotton/Elaston/Lycra/Spandex Yarn in various Ratios/Blends & Counts | 1 kg | 1 |  | Poly/Cotton Yarn in various ratios/Blends/Counts AND/OR | 1.045 kg |
|  |  |  | 1 |  | Elaston/Lycra/Spendex Yarn | 0.05 kg |
|  |  |  | 2 |  | Disperse Dyes of 100% Strength | 0.02 kg/kg content of Polyester in export. |
|  |  |  | 3 |  | Softening Agent | 0.01 kg |
|  |  |  | 4 |  | Optical whitener | 0.005 kg |
|  |  |  | 5 |  | Rangolite | 0.01 kg |
| J53 | Knitted Single Side Fleece Dyed & Printed Fabric containing Polyester & Other Blended Fabrics of different Materials and different Percentage | 1 kg | 1 |  | Relevant Polyester & Other Blended Knitted Fabrics of different percentage & different materials (Grey) | 1.05 kg |
|  |  |  | 2 |  | Disperse Dye (100% Strength) | 0.04 kg/kg of Polyester contents in the export product. |
|  |  |  | 3 |  | Softening Agent | 0.01 kg |
|  |  |  | 4 |  | Fleecing Agent | 0.005 kg |
|  |  |  | 5 |  | Optical Whitener | 0.005 kg |
|  |  |  | 6 |  | Rangolite | 0.01 kg |
| J54 | Knitted Single Side Fleece Dyed Fabric containing Polyester & Other Blended Fabrics of different Materials & different percentage | 1 kg | 1 |  | Polyester &Other Blended Knitted Fabrics of different percentage & different materials (Grey) OR | 1.045 kg |
|  |  |  |  |  | Dyed Polyester Texturised Yarn OR Dyed Blended Spun Yarn containing Polyester and other materials | 1.06 kg/kg content in the export product. 1.07 kg/kg content in the export product |
|  |  |  | 2 |  | Disperse Dye (100% Strength) | 0.02 kg/kg of Polyester content in the export product. |
|  |  |  | 3 |  | Softening Agent (100% Strength) | 0.01 kg |
|  |  |  | 4 |  | Fleecing Agent | 0.005 kg |
|  |  |  | 5 |  | Optical Whitener | 0.005 kg |
| Note.1 Items Nos.2 & 5 will be allowed only if the import item allowed is Grey Fabrics. |  |  |  |  |  |  |
| J55 | Knitted single side fleece grey fabric containing polyester and cotton in various blends and counts | 1 kg. | 1 |  | Polyester/ Cotton yarn in various blends/counts | 1.095 kg/kg of Polyester/ Cotton content in export product |
|  |  |  | 2 |  | Fleecing Lubricating Agent | 0.005 kg |
| J56 | Non-woven fusible coated interlining | 1 kg | 1 |  | Manmade Staple Fibres(polyester/Viscose) | 550 gms. |
|  |  |  | 2 |  | Acrylic Binder with 45% Solid content | 464 gms. |
|  |  |  | 3 |  | Coating Powder (Polymer of ethylene in Primary from LDP or HDP) | 305 gms. |
| J57 | Non-woven Inter- lining made of Man made Fibres containing 70% relevant Man made Fibre and 30% Acrylic Binder. | 1 kg | 1 |  | Relevant Man Made Fibres | 0.742 kg |
|  |  |  | 2 |  | Liquid Acrylic Binder (with 45% solid content) | 0.721 kg |
| J58 | Nylon sewing thread | 1 Kg | 1 |  | Nylon Filament Yarn | 1.02 Kg./Kg.content in export. |
| J59 | Polyester Cotton Combed Knitted Dyed fabrics | 1 Kg | 1 |  | Polyester Staple Fibre | 1.177 Kg/kg content in export |
| J60 | Polyester Dyed Knitted Fabric containing Elaston/Lycra/Spandex Yarn | 1 kg | 1 |  | 100% Spun Polyester Yarn | 1.10 kg/kg of spun Polyester Materials (Yarn) contents in the export product. |
|  |  |  | 2 |  | Elaston/Lycra/Spandex Yarn | 1.05 kg/kg of materials content in the export product. |
|  |  |  | 3 |  | POY/PFY | 1.10 kg/kg of Polyester Texturised Yarn contents in the export product. |
|  |  |  | 4 |  | Disperse Dyes (100% Strength) | 0.02 kg/kg of Polyester contents in the export product |
|  |  |  | 5 |  | Softening Agent | 0.01 kg/kg of export product. |
|  |  |  | 6 |  | Optical Whitener | 0.005 kg/kg of Polyester Materials contents in the export product. |
| J61 | Polyester dyed sewing thread | 1.0 Kg | 1 |  | 100% Polyester grey yarn | 1.02 Kg |
| J62 | Polyester dyed sewing thread | 1.0 Kg | 1 |  | 100% Polyester dyed yarn | 1.01 Kg |
| J63 | Polyester Texturised Filament Yarn (Dyed) | 1 kg | 1 |  | Partially Oriented Yarn (Dyed) | 1.01 kg |
| J64 | Polyester Viscose Texturised Blended Yarn | 1 kg | 1 |  | Polyster Staple Fibre | 1.05 kg/kg content in the export |
|  |  |  | 2 |  | Viscose Staple Fibre | 1.05 kg/kg content in export. |
|  |  |  | 3 |  | Polyester Filament Yarn | 1.02 kg/kg in export. |
| J65 | Polyester wadding/Non woven Polyester Interlining. | 1 Kg | 1 |  | Polyester Fibre | 0.836 Kg |
|  |  |  | 2 |  | Liquid Acrylic binder (with 45 % solid content) | 0.400 Kg |
| J66 | Polyester Wool Blended Fabrics | 1 kg | 1 |  | Wool Tops OR | 1.19 kg/kg content in the export product |
|  |  |  |  |  | Scoured Wool | 1.31 kg/kg of wool content in export product |
|  |  |  | 2 |  | Polyester Tops | 1.07 kg/kg of Polyester content in the export product |
| J67 | Polyester/Cotton blended Dyed spun Yarn. | 1 Kg | 1 |  | Polyester staple Fibre | 1.127 Kg/kg content in export |
|  |  |  | 2 |  | Disperse Dyes (100% Strength) | 0.02 Kg. |
| J68 | Polyester/Cotton Blended Grey Spun Yarn. | 1 Kg | 1 |  | Polyester Staple Fibre | 1.127 Kg./Kg. content in export. |
|  |  |  | 2 |  | Raw Cotton | 1.22 kg/kg content in the export product |
| J69 | Polyester/Texturised/Filament/Spun texturised Shirtings/Suitings/Dress material/Fabrics | 1.0 Kg | 1 |  | Partially Oriented Yarn OR | 1.10 kg./Kg. content in export. |
|  |  |  | 1 |  | Polyester Filament Yarn | 1.10 Kg./Kg content in export |
|  |  |  | 2 |  | Polyester Staple Fibre | 1.115 Kg./Kg. content in export. |
|  |  |  | 3 |  | Viscose Staple Fibre | 1.115 Kg./Kg content in export |
|  |  |  | 4 |  | Optical whitening Agent | 0.005 Kg. |
|  |  |  | 5 |  | Disperse dyes of 100% Strength | 2% of Polyester content in export |
|  |  |  | 6 |  | Vat Dyes of 100% strength | 0.015 Kg. |
| J70 | Polyster Staple Fibre | 1 kg | 1 |  | Polyster Chips | 1.03 kg |
|  |  |  | 2 |  | Spin Finish Oil | 0.01 kg |
| J71 | Polyster Viscose Blended Fabrics(Grey) | 1 kg | 1 |  | Polyester Viscose Blended Yarn | 1.035 kg/kg content in the export product |
|  |  |  | 2 |  | Polyvinyl Alcohol | 0.01 kg/kg content in the export product |
| J72 | Printed Acrylic Blankets/ Acrylic Blankets | 1 kg | 1 |  | Acrylic Fibre/Tow/Tow Waste OR Acrylic Yarn | 1.24 kg/kg content in the export product 1.15 kg/kg content in the export product |
|  |  |  | 2 |  | Cationic Dyes of 100% strength (only for printed export product) | 2% of Acrylic content in export. |
| J73 | Printed Fabrics/ Sarees/ made ups/ Hosiery made from 100% Polyester filament Yarn/ texturised yarn | 1 kg | 1 |  | Polyester filament Yarn OR | 1.10 kg./ kg content in export |
|  |  |  |  |  | Partially Oriented Yarn | 1.10 kg/ kg content in export |
|  |  |  | 2 |  | Polyvinyl Alcohol* | 0.010 kg. |
|  |  |  | 3 |  | Lubricating agent | 0.002 kg. |
|  |  |  | 4 |  | Antistatic agent | 0.001 kg. |
|  |  |  | 5 |  | Sodium Sulphoxylate formaldehyde OR | 0.010 kg. |
|  |  |  |  |  | Zinc Sulphoxylate formaldehyde (e.g. Rangolite DS/DP/C, jaykaylite, Safoline etc.) | 0.010 kg. |
| * Item 2 will not be allowed in case of hosiery products |  |  |  |  |  |  |
| J74 | Processed Knitted Fabrics containing Polyester-Viscose (Dyed/Printed) | 1 kg | 1 |  | Relevant Polyester-Viscose Knitted Grey Fabric | a)1.02 kg/kg content of only Dyed variety in export product.b)1.03 kg/kg content of only Printed variety in export product.c)1.05 kg/kg content of both Dyed and Printed variety in export product |
|  |  |  | 2 |  | Disperse Dyes (100% Strength) | 2% of weight of Polyester content in export product. |
|  |  |  | 3 |  | Vat/Reactive Dyes (100% Strength) | 1.5% of weight of Viscose content in export product. |
|  |  |  | 4 |  | Softening Agent | 0.01 kg/kg of export product. |
| J75 | Processed Knitted Fabrics containing Polyester Viscose Yarn(Dyed/Printed) | 1 kg | 1 |  | Polyester Viscose Yarn of relevant blend | a)1.045 kg/kg content of only Dyed variety in export product. b)1.055 kg/kg content of only Printed variety in export product. c)1.075 kg/kg content of both Dyed and Printed. |
|  |  |  | 2 |  | Disperse Dyes (100% Strength) | 2% of weight of Polyester content in export product. |
|  |  |  | 3 |  | Vat/Reactive Dyes (100% Strength) | 1.5% of weight of Viscose content in export product. |
|  |  |  | 4 |  | Optical Whitening Agent | 0.005 kg |
|  |  |  | 5 |  | Softening Agent | 0.01 kg |
| J76 | Processed Knitted Fabrics containing Polyester-Cotton (Dyed/Printed) | 1 kg | 1 |  | Relevant Polyester-Cotton Knitted Grey Fabric | a)1.02 kg/kg content of only Dyed variety in export product.b)1.03 kg/kg content of only Printed variety in export product.c)1.05 kg/kg content of both Dyed and Printed variety in export product |
|  |  |  | 2 |  | Disperse Dyes (100% Strength) | 2% of weight of Polyester content in export product. |
|  |  |  | 3 |  | Vat/Reactive Dyes (100% Strength) | 1.5% of weight of Cotton content in export product. |
|  |  |  | 4 |  | Optical Whitening Agent | 0.005 kg |
|  |  |  | 5 |  | Softening Agent | 0.01 kg |
| J77 | Processed Knitted Fabrics containing Polyester-Cotton Yarn(Dyed/Printed) | 1 kg | 1 |  | Polyester-Cotton Yarn of relevant blend | a)1.045kg/kg content of only Dyed variety in export product.b)1.055kg/kg content of only Printed variety in export product.c)1.075kg/kg content of both Dyed and Printed variety in export product |
|  |  |  | 2 |  | Disperse Dyes (100% Strength) | 2% of weight of Polyester content in export product. |
|  |  |  | 3 |  | Vat/Reactive Dyes (100% Strength) | 1.5% of weight of Cotton content in export product. |
|  |  |  | 4 |  | Optical Whitening Agent | 0.005 kg |
|  |  |  | 5 |  | Softening Agent | 0.01 kg |
| J78 | Quilted Lining with Back Lining | 1 sq.mtr. | 1 |  | Fabric to be used in the quilted lining | 1.02 sq.mtr. |
|  |  |  | 2 |  | Polyester Wadding to be used in quilted lining | 1.02 sq.mtr. |
| J79 | Rayon viscose Filament yarn | 1 kg | 1 |  | Rayon Grade Wood Pulp | 1.05 kg |
|  |  |  | 2 |  | Caustic Soda Lye | 0.65 kg |
|  |  |  | 3 |  | Electrolytic Zinc | 0.009 Kg |
|  |  |  | 4 |  | Carbon Disulphide | 0.3 Kg |
|  |  |  | 5 |  | Coning Oil | 0.007 Kg |
|  |  |  | 6 |  | Titanium Dioxide | 0.004 Kg |
|  |  |  | 7 |  | Coal-D-Type | 4.4 kg |
| Note: 1. Coal is allowed only under actual user condition. It is applicable to units having facilities for captive power generation using Coal. |  |  |  |  |  |  |
| J80 | Shoddy Synthetic Yarn | 1 kg | 1 |  | Synthetic Rags | 1.40 kg./kg. content in the export product. |
|  |  |  | 2 |  | Synthetic Waste | 1.05 kg./kg. content in the export product. |
|  |  |  | 3 |  | Synthetic Clipping | 1.15 kg/kg content in the export product |
|  |  |  | 4 |  | Pulled or garnetted Fibres made out of Woollen and/or Synthetic Rags/ Clippings/ Hard waste | 1.05 kg/kg content in the export product |
| J81 | Single side Fleece knitted Grey Dyed/ Printed Fabrics containing Polyester and Cotton Blends | 1 kg. | 1 |  | Polyester Cotton Blended Yarn | 1.12 kg of Polyester Cotton Blended Yarn per kg of relevant material content in the export product. |
|  |  |  | 2 |  | Disperse Dyes of 100% strength | 0.02 kg/kg weight of polyester content in the export product for dyed and 0.04 kg/kg for dyed and printed material. |
|  |  |  | 3 |  | Softening Agent | 0.01 kg. |
|  |  |  | 4 |  | Optical Whitener (for white fabric only) | 0.005 kg. |
|  |  |  | 5 |  | Fleecing Agent | 0.005 kg. |
| J82 | Sliver/ Carded Webs including cut forms made from Synthetic Waste | 1.0 Kg | 1 |  | Synthetic Waste | 1.02 Kg/kg content in export |
| J83 | Textile Bags | 1 kg. | 1 |  | Relevant Fabric and Lining | 1.02 kg/kg content in export product. |
| J84 | Dyed/Grey Twisted Polyester Texturised Yarn/Polyester Filament Yarn. | 1 kg | 1 |  | Polyester Texturised Yarn/Polyester Filament Yarn | 1.01 kg/kg content in the export product |
|  |  |  | 2 |  | Disperse Dyes (100% Strength) | 0.02 kg |
| Note: Item 2 will be allowed only in case of export of Dyed Yarn. |  |  |  |  |  |  |
| J85 | Velvet Fabrics duly Embroidered with Glass/Plastic/Metallic Beads & Tikki | 1 kg | 1 |  | Velvet Fabric | 1.01 kg/kg content in the export product |
| J86 | Velvet Hair Band | 1 kg | 1 |  | Velvet Fabric | 1.01 kg |
| J87 | Viscose/ Cotton Blended Yarn | 1.0 Kgs. | 1 |  | Viscose Fibre Suitable for Spinning | 1.05 Kg/Kg of viscose content. |
| J88 | White Ground Brasso printed Saree Mercerised Polyester/Cotton blended | 1.0 Kg | 1 |  | Polyester Fibre | 1.21 Kg./Kg. content in export. |
|  |  |  | 2 |  | Polyvinly Alcohol | 1% of polyester content. |
|  |  |  | 3 |  | Disperse Dyes of 100% strength | 2% of polyester content. |
| J89 | Worsted Spun Yarn(Grey)containing Polyester and Wool | 1 Kg. | 1 |  | Polyester Fibre OR | 1.13 kg/kg content in export |
|  |  |  | 1 |  | Polyester Tow | 1.08 kg/kg content in export |
|  |  |  | 2 |  | Wool Tops | 1.10 kg/Kg content in export |
|  |  |  | 3 |  | Tinting Agent | 0.005 kg. |
| J90 | Woven Jacquard Upholstery | 1 kg | 1 |  | Polyester Texturised Yarn | 1.10 kg/kg of relevant Polyester Texturised Yarn content in the export product. |
| J91 | 100% Cotton Bleached/ Dyed/Printed Fabrics/ Bed Sheets Sets with/ without Pillow and other Made ups | 1 Kg | 1 |  | Grey Cotton Fabric | 1.05 Kg. |
|  |  |  | 2 |  | Azoic/Sulphur/Vat/ Reactive Dyes of 100% strength | 0.015 Kg. |
|  |  |  | 3 |  | Hydrogen Peroxide | 0.020 Kg. |
|  |  |  | 4 |  | Silicon Emulsion | 0.010 Kg. |
|  |  |  | 5 |  | Titanium Dioxide | 0.010 Kg. |
|  |  |  | 6 |  | Bronze Powder (for Bronze printed goods) | 0.010 Kg. |
|  |  |  | 7 |  | Optical Whitening Agent (only for white or printed goods having white ground) | 0.005 kg |
| J92 | Cotton fabric - dyed with/without elaston/lycra/spandex yarn | 1 kg | 1 |  | Cotton Yarn OR Raw cotton for carded counts of yarn of 40s & below OR Raw cotton for combed counts of yarn of 40s and below OR Raw cotton for combed counts of yarn of above 40s | 1.10 kg 1.225 kg/kg content in the export product 1.289 kg/kg content in the export product 1.325 kg/kg content in the export product |
|  |  |  | 2 |  | Polyvinyl Alcohol (PVA) Sizing Material | 0.01 kg |
|  |  |  | 3 |  | Sodium Hydro Sulphite | 0.02 kg |
|  |  |  | 4 |  | Vat/ Reactive/ Sulphur/ Pigment/ Dyes (100% strength) | 0.015 kg/kg of cotton content in export product. |
|  |  |  | 5 |  | Elaston / lycra / spandex yarn OR Stretch yarn (mixed yarn containing 95% cotton and 5% of elaston/lycra/spandex yarn) | 1.05 kg/kg of materials content in the export product 1.05 kg/kg content in the export product |
|  |  |  | 6 |  | Fuel for power generation Naphtha OR | 5.50 kg |
|  |  |  |  |  | Furnace Oil OR | 4.10 litres |
|  |  |  |  |  | LNG | 3.44 kg |
| Note: 1. Naphtha, Furnace Oil and LNG shall be allowed only under AU condition. Note: 2. The above norms for fuels are applicable to units having spinning, weaving and processing activities or only weaving and processing activities as the case may be. Also the said units should have facilities for captive power generation using naptha/furnace oil/LNG. |  |  |  |  |  |  |
| J93 | 100% Cotton Fabric - Grey | 1 kg | 1 |  | Polyvinyl Alcohol | 0.01 kg |
|  |  |  | 2 |  | 100% Cotton Yarn OR Raw cotton for carded counts of yarn of 40s & below OR Raw cotton for combed counts of yarn of 40s and below OR Raw cotton for combed counts of yarn of above 40s | 1.05 Kg 1.175 kg/kg content in the export product 1.270 kg/kg content in the export product 1.290 kg/kg content in the export product |
| J94 | 100% Cotton Fabric - Printed | 1 kg | 1 |  | Cotton Yarn OR Raw cotton for carded counts of yarn of 40s & below OR Raw cotton for combed counts of yarn of 40s and below OR Raw cotton for combed counts of yarn of above 40s | 1.10 kg 1.225 kg/kg content in the export product 1.289 kg/kg content in the export product 1.325 kg/kg content in the export product |
|  |  |  | 2 |  | Polyvinyl Alcohol (PVA) Sizing Materials | 0.01 kg |
|  |  |  | 3 |  | Sodium Hydro Sulphite | 0.02 kg |
|  |  |  | 4 |  | Vat Dyes/Reactive Dyes (100% Strength) | 0.015 kg |
| J95 | 100% Cotton Fabrics - Bleached | 1 kg | 1 |  | Polyvinyl Alcohol | 0.01 kg |
|  |  |  | 2 |  | Hydrogen Peroxide | 0.02 kg |
|  |  |  | 3 |  | Raw cotton for carded counts of yarn of 40s & below OR Raw cotton for combed counts of yarn of 40s and below OR Raw cotton for combed counts of yarn of above 40s | 1.225 kg/kg content in the export product 1.289 kg/kg content in the export product 1.325 kg/kg content in the export product |
| J96 | Cotton Dyed Denim Cloth with/without over-dyeing/printing/coating containing 50% or more by weight of Cotton | 1 kg. | 1 |  | Raw Cotton OR Cotton Yarn AND/OR Cotton blended yarn with Manmade yarn (excluding spandex / lycra yarn) Polyester / Viscose / Acrylic / Nylon / Tencel (Lyocell Fibre) AND/OR Manmade Yarn (excluding spandex/ lycra yarn) Acrylic/ Polyester / Nylon/ Tencel (Lyocell Fibre) | 1.24 kg./kg. content in export product 1.04 kg/kg content in the export product. 1.05 kg/kg content in the export product. 1.05 kg/kg content in the export product. |
|  |  |  | 2 |  | Vat Indigo Blue Dye OR Vat Indigo Blue solution Dye 40% strengh OR Vat (other than indigo) / Reactive / Sulphur / Pigment / dyes | 0.05 kg/kg of cotton content in the export product 0.125 kg/kg of cotton content in export product 0.015 kg/kg of cotton content in export product |
|  |  |  | 3 |  | Polyvinyl Alcohol | 0.03 kg. |
|  |  |  | 4 |  | Sodium Hydrosulphite | 0.02 kg. |
|  |  |  | 5 |  | Caustic Soda Flakes | 0.02 kg. |
|  |  |  | 6 |  | Elaston/ Lycra/ Spandex yarn OR Stretch Yarn (mixed yarn containing 90% or more cotton and upto 10% of Elaston/ Lycra/ Spandex Yarn) | 1.05 kg/kg of material content in the export product 1.05 kg/kg of materials content in the export product |
|  |  |  | 7 |  | Poly Acrylic Synthetic Sizing agent/ Poly Acryl Amide Solution | 0.016 kg |
|  |  |  | 8 |  | Sulfoesterifiedamide Starch/ modified starch | 0.044 kg |
|  |  |  | 9 |  | (a) Fuel for power generation for mills having spinning, weaving and processing activities (raw cotton as raw material) Naphtha OR SKO OR Furnace Oil OR HSD OR LNG OR Coal | 1.22 kg 1.22 kg 0.92 Litre 1.10 Litre 0.77 kg 2.18 kg |
|  |  |  |  |  | (b) Fuel for power generation for mills having weaving and processing activities (cotton yarn as raw material) Naptha OR SKO OR Furnace Oil OR HSD OR LNG OR Coal | 0.73 kgs 0.73 kgs 0.65 Litre 0.77 Litre 0.55 kg 1.30 kg |
| Note: 1. Naphtha, SKO, Furnace Oil, HSD, LNG and Coal shall be allowed only under AU condition. Note: 2. The above norms for fuel are applicable to units having spinning, weaving and processing or only weaving and processing activities as the case may be. Also the said units should have facilities for captive power generation using naptha/SKO/Furnace Oil/HSD/LNG/Coal. |  |  |  |  |  |  |
| J97 | Bleached & Dyed Cotton Knitted Fabrics | 1 kg | 1 |  | Cotton Yarn Grey OR Knitted Cotton Fabric (Grey) | 1.05 kg 1.025 kg |
|  |  |  | 2 |  | Dyes (Vat/Sulphur) (100% Strength) | 1.5% of weight of Dyed Fabric. |
|  |  |  | 3 |  | Hydrogen Peroxide | 2% of weight of Fabric. |
|  |  |  | 4 |  | Optical Whitening Agent | 0.5% of wt. of Fabric. |
| J98 | Bleached/Dyed/Printed Cotton Knitted Fabrics with/without Elaston/ Lycra/ Spandex Yarn | 1 kg | 1 |  | 100% Cotton Yarn | 1.10 kg/kg of Cotton materials content in the export product. |
|  |  |  | 2 |  | Elaston/Lycra/Spandex Yarn | 1.05 kg/kg of Materials content in the export products. |
|  |  |  | 3 |  | Dyes(Vat/Sulphur/Reactive) of 100% Strength | 0.015 kg/kg of Cotton Material content in the export product. |
|  |  |  | 4 |  | Silicon Emulsion (Not to be allowed if the export product is dyed or printed) | 0.01 kg/kg of Cotton Materials contents in the export product. |
|  |  |  | 5 |  | Optical whitener | 0.005 kg/kg of Cotton Materials in the export product. |
|  |  |  | 6 |  | Hydrogen Peroxide (For white or printed goods having white ground) | 0.02 kg/kg of Cotton Materials content in the export product. |
| Note: 1. In respect of Sl.Nos. 113 to 115 below, 1% additional wastage may be allowed in case of Plied Yarn. |  |  |  |  |  |  |
| J99 | Carded yarn of counts 40s and below | 1 kg | 1 |  | Raw Cotton | 1.125 kg |
|  |  |  | 2 |  | Fuel for power generation HSD OR Furnace Oil | 1.15 litres 1 litre |
| Note: 1. HSD and Furnace Oil shall be allowed only under AU condition. Note: 2. The above norms for fuel are applicable only to those units having facilities for captive power generation using HSD/Furnace Oil. |  |  |  |  |  |  |
| J100 | Combed Cotton/Melange Yarn of counts 40s and below | 1 kg | 1 |  | Raw Cotton | 1.40 kg |
|  |  |  | 2 |  | Fuel for power generation HSD OR Furnace Oil | 1.50 litres 1.20 litres |
| Note: 1. HSD and Furnace Oil shall be allowed only under AU condition. Note: 2. The above norms for fuel are applicable only to those units having facilities for captive power generation using HSD/Furnace Oil. |  |  |  |  |  |  |
| J101 | Combed Cotton/Melange Yarn of counts above 40s | 1 kg | 1 |  | Raw Cotton | 1.40 kg |
|  |  |  | 2 |  | Fuel for power generation HSD OR Furnace Oil | 1.74 litres 1.45 litres |
| Note: 1. HSD and Furnace Oil shall be allowed only under AU condition. Note: 2. The above norms for fuel are applicable only to those units having facilities for captive power generation using HSD/Furnace Oil. |  |  |  |  |  |  |
| J102 | Cotton Bags | 1 No | 1 |  | Rings | Net to Net |
|  |  |  | 2 |  | End Tips | Net to Net |
|  |  |  | 3 |  | Push Lock | Net to Net |
|  |  |  | 4 |  | Buckles | Net to Net |
|  |  |  | 5 |  | Dog Hooks | Net to Net |
| J103 | Cotton Fabric Dyed and printed | 1 Kg | 1 |  | Cotton Yarn OR Raw cotton for carded counts of yarn of 40s & below OR Raw cotton for combed counts of yarn of 40s and below OR Raw cotton for combed counts of yarn of above 40s | 1.10 kg/kg content in export 1.225 kg/kg content in the export product 1.289 kg/kg content in the export product 1.325 kg/kg content in the export product |
|  |  |  | 2 |  | Dyes (Vat Dyes) of 100% strength (for Dyed material) | 0.015 kg |
|  |  |  | 3 |  | Pigments (for printing) | 0.01 kg |
|  |  |  | 4 |  | Titanium Dioxide | 1% of printed fabric |
|  |  |  | 5 |  | Hydrogen Peroxide | 2% of the wt.of cotton |
| J104 | Cotton Knitted Fabric (Bleached/Dyed/ Printed) | 1 Kg | 1 |  | Cotton Yarn OR | 1.10 kg/kg content in export. |
|  |  |  |  |  | Raw Cotton | 1.25 kg/kg content in export |
|  |  |  | 2 |  | Dyes (Vat/ Sulphur) | 1.5% wt. of fabric. |
|  |  |  | 3 |  | Titanium Dioxide | 1% of printed fabric weight |
|  |  |  | 4 |  | Bronze Powder (for Bronze printed goods) | 1% of wt. of fabric. |
|  |  |  | 5 |  | Hydrogen Peroxide | 2%of the wt.of fabric |
|  |  |  | 6 |  | Optical Whitening Agent (for white or printed fabric having white ground) | 1/2 % of the wt.of fabric |
| J105 | Cotton knitted Fabric with Elaston/Lycra/Spandex Yarn | 1 kg | 1 |  | Spandex Yarn | 1.03 kg/kg content in the export product. |
|  |  |  | 2 |  | Cotton Yarn OR Raw cotton for carded counts of yarn of 40s & below OR Raw cotton for combed counts of yarn of 40s and below OR Raw cotton for combed counts of yarn of above 40s | 1.03 kg/kg content in the export product 1.225 kg/kg content in the export product 1.289 kg/kg content in the export product 1.325 kg/kg content in the export product |
|  |  |  | 3 |  | Needles | 50 Nos./ MT of Fabric export. |
|  |  |  | 4 |  | Sinkers | 25 Nos./ MT of Fabric export. |
| Note: The CIF value of items 3 & 4 taken together shall not exceed 1% of FOB value of exports. |  |  |  |  |  |  |
| J106 | Cotton Knitted Fabrics(Grey) | 1 kg | 1 |  | Cotton Yarn | 1.03 kg/kg content in the export product. |
| J107 | Cotton Rami Blended knitted Sweaters | 1 kg | 1 |  | Cotton Rami Blended Yarn | 1.15 kgs. |
| Note: Blend of Cotton Rami should conform in import and export. |  |  |  |  |  |  |
| J108 | Dyed 100% Cotton Table Cover | 1 kg | 1 |  | Cotton Yarn (Relevant Quality) | 1.05 kg/kg content in the export product. |
|  |  |  | 2 |  | Reactive Dyes (100% Strength) | 0.015 kg |
| J109 | Dyed and/or Printed Cotton made-ups | 1 Kg | 1 |  | Cotton fabric | 1.05 Kg. per kg. content in export. |
|  |  |  | 2 |  | Sodium Hydro Sulphite | 2% of weight of fabrics allowed. |
| J110 | Dyed Chindi Cotton Durry | 1 kg. | 1 |  | Pieces of fabrics containing cotton | 1.05 kg/kg. content in export. |
|  |  |  | 2 |  | Cotton yarn | 1.05 kg/kg. content in export. |
|  |  |  | 3 |  | Reactive Dyes of 100% strength | 1.5% of weight of export. |
| J111 | Dyed Cotton Bath Rugs | 1 kg | 1 |  | Cotton Yarn | 1.05 kg/kg content in export. |
|  |  |  | 2 |  | Cotton Fabrics | 1.02 kg/kg content in export |
|  |  |  | 3 |  | Reactive Dyes of 100% Strength | 0.015 kg |
| J112 | Dyed Cotton Durries | 1 kg | 1 |  | 100% Cotton Coarse Yarn (Below 10 Counts) | 1.05 kg/kg content in the export product. |
|  |  |  | 2 |  | Reactive Dyes (100% Strength) | 0.015 kg/kg of cotton yarn content in the export product. |
| J113 | Flax yarn | 1 kg. | 1 |  | Flax fibre | 1.66 kg |
| J114 | Grey Fabric made out of Carded Yarn of counts below 40s | 1 kg | 1 |  | Raw Cotton | 1.15 kgs. |
| J115 | Grey fabrics made out of combed yarn of counts below 40s | 1 kg. | 1 |  | Raw Cotton | 1.22 kg |
| J116 | Grey Fabrics manufactured out of combed yarn above 40s | 1 kg | 1 |  | Raw cotton | 1. 26 kg |
| J117 | Knitted Bleached/ Dyed/Printed Cotton Fabric | 1 kg | 1 |  | Grey Knitted Cotton Fabric | 1.050 kgs |
|  |  |  | 2 |  | Hydrogen Peroxide | 2% of weight of Fabric |
|  |  |  | 3 |  | Optical Whitening Agent | 0.5% of weight of Fabric. |
|  |  |  | 4 |  | Dyes (Vat/Sulphur Reactive) of 100% strength (only for dyed/printed knitted fabric) | 0.015 kg |
|  |  |  | 5 |  | Bronze Powder (for Bronze printed goods only) | 1% of wt. of fabric. |
| J118 | Deleted |  |  |  |  |  |
| J119 | Processed Cotton Made-ups other than Grey | 1 kg | 1 |  | Processed Cotton Fabrics | 1.02 kg/kg content in the export product |
| J120 | Processed fabrics made out of combed Yarn of counts below 40s | 1 kg. | 1 |  | Raw Cotton | 1.289 kg |
| J121 | Processed Fabrics manufactured out of combed yarn above 40s | 1 kg. | 1 |  | Raw Cotton | 1.325 kg |
| J122 | Ramie/Cotton Yarn | 1 kg | 1 |  | Ramie Fibre | 1.14 kg/kg of Ramie Fibre content in the export product. |
| J123 | 100% Natural Silk (Chiffon/Chinnon/Crepe/Georgette/Taffeta/Satin) Fabrics/Made-ups/Sarees | 1 kg | 1 |  | Mulberry Raw Silk (Gummed) of any grade (other than Dupion Yarn) | 1.39 kg/kg content in the export product |
|  |  |  | 2 |  | Dupion Silk Yarn (gummed and untwisted) OR Dupion Silk Yarn (de-gummed and twisted) | 1.39 kg/kg content in export product 1.05 kg/kg content in export product |
| J124 | 100% Natural Silk Fabrics/made-ups/sarees (other than those covered by Sl.No. J.123 above)/garments containing one or more than one specified constituent raw silk/silk yarn. | 1 kg | 1 |  | Mulberry Raw Silk of any grade (other than Dupion Yarn) | 1.39 kg/kg content in export product |
|  |  |  | 2 |  | Dupion Silk Yarn (gummed and untwisted) OR Dupion Silk Yarn (degummed and twisted) | 1.39 kg/kg content in export product 1.05 kg/kg content in the export product |
|  |  |  | 3 |  | Noil Silk Yarn | 1.05 kg/kg content in export product |
|  |  |  | 4 |  | Spun Silk Yarn | 1.05 kg/kg content in export product |
|  |  |  | 5 |  | Reeled Tussar Silk Yarn/ Tussar Silk Yarn | 1.15 kg/kg content in export product |
| Note: 1. In case of export of garments, item Nos. 3 & 4 will be allowed @ 1.07 kg/kg content in export product and item No. 5 will be allowed @ 1.17 kg/kg content in export product. Note: 2. In case of export of garments, if the garments are fitted with the Beads, Sequines, Chattons etc., then the Beads,Sequins, Chattons etc. will be allowed on "Net plus 1% wastage basis". Note: 3. In case of export of garments, if the lining material is to be/has been used in the silk garments, the same may be allowed on Net plus 5% wastage basis. |  |  |  |  |  |  |
| J125 | 100% Natural Silk Garments made of relevant Silk Fabric of relevant GSM with +/- 10% variation | 1 kg | 1 |  | 100% Natural relevant Silk fabric of relevant GSM with +/- 10% variation | 1.01 kg/kg content in export product |
| Note: 1. If the garments are fitted with the Beads, Sequins, Chattons, etc., then the Beads, Sequins, Chattons etc., will be allowed on "Net plus 1% wastage basis".Note: 2 If the lining material is to be/has been used in the Silk Garments, the same may be allowed on "Net plus 5% wastage basis". |  |  |  |  |  |  |
| J126 | Deleted |  |  |  |  |  |
| J127 | Deleted |  |  |  |  |  |
| J128 | Deleted |  |  |  |  |  |
| J129 | Natural silk mixed Fabrics/made-ups/sarees/garments containing one or more than one specified constituent silk yarn and any other Textile yarn | 1 kg | 1 |  | Mulberry Raw Silk of any grade (other than Dupion Yarn) | 1.39 kg/kg content in the export product |
|  |  |  | 2 |  | Dupion Silk Yarn (gummed and untwisted) OR Dupion Silk Yarn (degummed and twisted) | 1.39 kg/kg content in export product 1.05 kg/kg content in the export product |
|  |  |  | 3 |  | Noil Silk Yarn | 1.05 kg/kg content in export product |
|  |  |  | 4 |  | Spun Silk Yarn | 1.05 kg/kg content in export product |
|  |  |  | 5 |  | Reeled Tussar Silk Yarn/Tussar Silk Yarn | 1.15 kg/kg content in export product |
|  |  |  | 6 |  | Any other textile yarn | 1.05 kg/kg content in export product |
| Note: 1. In case of export of garments, item nos. 3,4 & 6 will be allowed @ 1.07 kg/kg content in export product and item no. 5 will be allowed @ 1.17 kg/kg content in export product Note: 2. In case of export of garments, if the garments are fitted with the Beads, Sequins, Chattons etc. then the Beads, Sequins, Chattons etc. will be allowed on "Net plus 1% wastage basis". Note: 3. In case of export of garments, if the lining material is to be/has been used in the silk garments, the same may be allowed on Net plus 5% wastage basis. |  |  |  |  |  |  |
| J130 | Deleted |  |  |  |  |  |
| J131 | Deleted |  |  |  |  |  |
| J132 | Silk Yarn | 1 kg | 1 |  | Silk Waste OR Silk Tops | 1.08 kg 1.05 kg |
| J133 | Deleted |  |  |  |  |  |
| J134 | Deleted |  |  |  |  |  |
| J135 | 100% Cotton Knitted Briefs | 1.0 Kg | 1 |  | Cotton Yarn OR | 1.15 kg/kg content in the export product. |
|  |  |  |  |  | Cotton knitted fabric | 1.05 kg/kg content in export |
|  |  |  | 2 |  | Rubber/Elastic Tape upto width 1 inch | 1.005 kg/kg content in the export product. |
| J136 | 100% Cotton Terry Towel (Bleached/Dyed) | 1 Kg | 1 |  | 100% Cotton/Combed Yarn | 1.10 Kg. |
|  |  |  | 2 |  | Hydrogen Peroxide | 0.02 kg |
|  |  |  | 3 |  | Softening Agent | 0.01 kg |
|  |  |  | 4 |  | Reactive or Direct or Vat or Sulphur Dyes of 100% strength | 0.015 kg |
|  |  |  | 5 |  | Optical Whitening Agent of 100% Strength | 0.005 kg |
| Note: Item Nos. 3 & 4 will be allowed only against export of Dyed Terry Towel. |  |  |  |  |  |  |
| J137 | Anoraks | 1 No. | 1 |  | PU/PVC coated Nylon/Polyester Fabric OR Any other relevant fabric | 3.20 Sq.Mtr. |
|  |  |  | 2 |  | Reflective Fabric in 2" width | Net+0.5% |
|  |  |  | 3 |  | PU/PVC Fusing/Sealing Tape in 20 mm width | Net+0.5% |
|  |  |  | 4 |  | Nylon Zippers | Net to Net |
|  |  |  | 5 |  | Metal Studs | Net to net. |
|  |  |  | 6 |  | Polyester Thread | 200 Mtrs. |
|  |  |  | 7 |  | Labels | Net to net |
| J138 | Articles of Hosiery/ Knitwears made of 100% Mohair/ 100% Wool | 1 kg | 1 |  | Scoured Wool OR Wool Tops/Mohair Tops | 1.35 kg 1.30 kg |
| J139 | Deleted |  |  |  |  |  |
| J140 | Articles of Hosiery/ knitwears made of 100% wool | 1 kg | 1 |  | Woollen Yarn/ Woollen Worsted Yarn not exceeding 32 BWS | 1.143 kg |
|  |  |  | 2 |  | Perchloro Ethylene | 0.05 kg |
| J141 | Articles of Hosiery/ knitwears made of woollen blended yarn | 1 kg | 1 |  | Synthetic Woollen Blended Yarn containing Synthetic Fibre like Acrylic/Polyester/Nylon of relevant blend. | 1.138 Kg/Kg content in the export product. |
|  |  |  | 2 |  | Perchloro ethylene | 0.05 Kg/Kg content of wool & other animal hair content in the export product |
| J142 | Men's /Ladies Bib and Brace | 1 No. | 1 |  | Relevant Fabrics | 1.92 Sq. mtr. |
| J143 | Deleted |  |  |  |  |  |
| J144 | Blouson with or without lining | 1 No. | 1 |  | Fabric to be used in the garment | 3.35 sq.m |
|  |  |  | 2 |  | Lining fabric to be used in the garment | 3.01 sq.m |
| Note: Item No.2 shall be allowed, if the export product is with lining |  |  |  |  |  |  |
| J145 | Blouson with lining & Hood | 1 No. | 1 |  | Fabric to be used in the garment | 3.5 sq.m |
|  |  |  | 2 |  | Lining fabric to be used in the garment | 3.15 sq.m |
| J146 | Blouson with lining and Non- Woven fusible lining | 1 No. | 1 |  | Fabric to be used in the garment | 3.35 sq.m |
|  |  |  | 2 |  | Lining fabric to be used in the garment | 3.01 sq.m |
|  |  |  | 3 |  | Non-woven fusible lining fabric used in the garment | 0.40 sq.m |
| J147 | Blouson with lining, Non-woven fusible lining & Hood | 1 No. | 1 |  | Fabric to be used in the garment | 3.5 sq.m |
|  |  |  | 2 |  | Lining fabric to be used in the garment | 3.15 sq.m |
|  |  |  | 3 |  | Non-woven fusible lining fabric used in the garment | 0.40 sq.m |
| J148 | Girls/Boys Shorts | 1 No | 1 |  | Relevant Fabric | 0.75 sq. mtr. |
| J149 | Children Nikars/Shorts | 1 No. | 1 |  | Relevant Fabric | 0.50 Sq.Mtr. |
| J150 | Children Pant/Trouser | 1 No. | 1 |  | Relevant Fabric | 0.85 Sq.Mtr. |
| J151 | Deleted |  |  |  |  |  |
| J152 | Deleted |  |  |  |  |  |
| J153 | Deleted |  |  |  |  |  |
| J154 | Children/Infants Cotton Denim Full Sleeves Jackets | 1 No. | 1 |  | Cotton Denim Fabric | 1.20 Sq.Mtr. |
| J155 | Cotton and Polyester mixed knitted readymade garments | 1 kg | 1 |  | Polyester Texturised yarn | 1.250 kg/ kg content in export |
|  |  |  | 2 |  | Disperse Dyes of 100% strength | 0.02 kg |
| J156 | Cotton garments made from Dyed, Printed Fabric | 1 kg | 1 |  | Cotton Fabric (Grey) | As per norm given hereafter for a relevant type of Garment. |
|  |  |  | 2 |  | Reactive Dyes of 100% strength | 0.020 kg |
|  |  |  | 3 |  | Titanium Dioxide | 0.010 kg |
|  |  |  | 4 |  | Bronze Powder | 0.010 kg |
| J157 | Cotton knitted Elastic cuff unbleached | 1 pair | 1 |  | Cotton yarn | 1.05 kg/kg content in export. |
|  |  |  | 2 |  | Rubber Thread | 1.52 gms. |
| J158 | Cotton knitted gloves-seamless | 1 pair | 1 |  | Cotton yarn | 1.05 kg/kg content in export. |
|  |  |  | 2 |  | Rubber Thread | 1.52 gms. |
| J159 | Cotton Knitted Ladies Panty/ Gents Brief (Dyed/Grey) | 1 kg. | 1 |  | Cotton yarn (grey) | 1.20 kg |
|  |  |  | 2 |  | Dyes of 100% strength (Vat/ Sulphur) (for dyed product only) | 2% of cotton content in the export product |
| J160 | Cotton knitted readymade garments | 1 kg. | 1 |  | Dyes (Vat/ Sulphur) of 100% Strength | 1.5% of export wt. |
| J161 | Knitted Sweaters | 1 kg | 1 |  | Relevant yarn (natural/manmade) | 1.10 kg/kg content in the export product |
| J162 | Cotton knitted with Elaston/Lycra/Spandex Readymade Garments Containing lycra OR Spandex OR Elaston yarn between 3-10% | 1 Kg | 1 |  | Cotton Yarn | 1.2 kg/kg content in the export product |
|  |  |  | 2 |  | Elaston OR Lycra OR Spandex Yarn | 1.1 kg/kg content in the export product |
| J163 | Cotton Socks/ Stockings (knitted) | 1 kg. | 1 |  | Cotton yarn | 1.05 kg. |
|  |  |  | 2 |  | Elastic yarn | 50 gm. for 1 kg cotton content |
| J164 | Cotton sports vest (knitted) | 1 kg. | 1 |  | Cotton yarn | 1.25 kg |
| J165 | Cotton Track Suit (Knitted) | 1 kg. | 1 |  | Cotton knitted fabric | 1.15 kg |
| J166 | Coverall (Industrial Worker Garment/ Boiler/ Work Suit) made out of Polyester/ Cotton Blended Fabric) | 1 No. (1.75 Mtr. Length +/-5%) | 1 |  | Polyester/ Cotton Blended Fabrics | 4.05 Sq. Mtr. |
| J167 | Dyed Acrylic Knitwear (including general cut items) | 1 kg | 1 |  | Acrylic Fibre OR | 1.30 kg/kg content in export. |
|  |  |  |  |  | Acrylic Yarn | 1.20 kg/kg content in export. |
|  |  |  | 2 |  | Cationic Dyes of 100% Strength | 0.020 kg |
| J168 | Dyed cotton knitted ladies night dress/gown | 1 No | 1 |  | Raw Cotton OR | 1.25 Kg/Kg. content in export. |
|  |  |  |  |  | Cotton Yarn (Grey) | 1.15 kg/kg content in export |
|  |  |  | 2 |  | Dyes (other than disperse, metal complex, acid dyes) | 1.5% of weight of export product. |
|  |  |  | 3 |  | Sodium Hydro Sulphite | 2% of weight of export product. |
|  |  |  | 4 |  | Silicon emulsion | 1% of weight of export product. |
|  |  |  | 5 |  | Optical whitening agent. | 0.5% of the weight of export poroduct. |
| J169 | Dyed Placemat, Napkin, Table Cloth, Cushion Cover and Comfort Cover etc. | 1 kg | 1 |  | Grey Cotton Fabric | 1.05 kg/kg content in export product. |
|  |  |  | 2 |  | Sulphur Dyes, Vat Dyes, Fibre Reactive Dyes(100% Strength) | 0.015 kg/kg content in the export product |
| J170 | Dyed Printed Salwar Kameez and Dupatta made of Polyester (Weight reduction process) | 1 kg | 1 |  | Polyester Filament Yarn/POY | 1.25 kg/kg content in export. |
|  |  |  | 2 |  | Polyvinyl Alcohol | 1% of weight of Polyester content in export. |
|  |  |  | 3 |  | Sodium Sulpho- xylate Formal- dehyde OR Zinc Sulphoxylate Formaldehyde (e.g. Rangolite DS/DP/C, Jayka- lite Salfoline etc.) | 0.010 kg 0.010 kg |
|  |  |  | 4 |  | Silicon Compound (Amino Silicon/ Emulson) | 0.020 kg |
|  |  |  | 5 |  | Optical Whitening Agent | 0.005 kg |
|  |  |  | 6 |  | Ionetrap (Leveling Agent for Disperse Dyes) | 0.010 kg |
|  |  |  | 7 |  | Disperse Dyes of 100% Strength | 4% of weight of Polyester content in export. |
|  |  |  | 8 |  | Lubricating Agent | 0.002 kg |
|  |  |  | 9 |  | Antistatic Agent | 0.001 kg |
|  |  |  | 10 |  | Printing Gum | 0.020 kg |
| J171 | Dyed/Printed Readymade Garments fitted with or without beads, sequins, chattons etc. and with or without lining | 1 kg | 1 |  | Fabric to be used in the garment | 1.1 kg/kg content in the export product |
|  |  |  | 2 |  | Beads, Sequins, Chattons etc. | Net +1% wastage |
|  |  |  | 3 |  | Lining to be used in the garments | Net + 5% wastage |
| Note: 1. Import item no. 2 shall be allowed, if the export is fitted with Beads, Sequins, Chattons etc.Note: 2. Import item no. 3 shall be allowed, if the export product is filled with lining material. |  |  |  |  |  |  |
| J172 | Dyed Wool Blended Knitwears containing wool, Acrylic & Polyester | 1 kg | 1 |  | Scoured Wool | 1.35 kg/kg of wool fibre contents in the export product. |
|  |  |  | 2 |  | Acrylic Fibre | 1.27 kg/kg of Acrylic fibre contents in the export product. |
|  |  |  | 3 |  | Polyester Fibre | 1.27 kg/kg of Polyester content in the export product |
|  |  |  | 4 |  | Acid Dyes (100% Strength) | 0.015 kg/kg of Wool contents in the export product. |
|  |  |  | 5 |  | Disperse Dye (100% Strength) | 0.02 kg/kg of Polyester contents in the export product. |
|  |  |  | 6 |  | Cationic Dyes (100% Strength) | 0.02 kg/kg of Acrylic fibre contents in the export product. |
| J173 | Gents Bermudas | 1 No | 1 |  | Relevant Fabric | 1.00 Sqr.Mtr. |
| J174 | Deleted |  |  |  |  |  |
| J175 | Gents Half sleeve shirts | 1 No. | 1 |  | Fabric to be used in the garment | 2.15 sq.m |
| J176 | Gents Jacket with lining | 1 No. | 1 |  | Fabric to be used in the garment | 2.48 sq.m |
|  |  |  | 2 |  | Lining fabric used in the garment | 1.86 sq.m |
| J177 | Gents Long Full sleeve Jacket with Hood & Collar without Lining (Made out of Nylon Tafetta Fabric) | 1 pc. | 1 |  | Nylon Tafetta Fabric (GSM=60 +/- 10%) | 3.00 Sq. Mtr. |
| J178 | Gents Jackets and overcoating made out of Blanketing Cloth with lining | 1 kg | 1 |  | Synthetic Waste | 1.13 kg/kg content in export product. |
|  |  |  | 2 |  | Wool Waste | 1.38 kg/kg content in export product. |
|  |  |  | 3 |  | Scoured Wool | 1.38 kg/kg content in export product. |
|  |  |  | 4 |  | Woollen Rags/ Synthetic Rags | 1.74 kg/kg content in export product. |
|  |  |  | 5 |  | Lining Fabric | 1.86 Sq. Mtrs. |
|  |  |  | 6 |  | Polyester Wadding | Net to net. |
| J179 | Gents Long Coat with Lining | 1 No. | 1 |  | Fabric to be used in the garment | 5.00 sq.m |
|  |  |  | 2 |  | Lining Fabric to be used in the garment | 4.5 sq.m |
|  |  |  | 3 |  | Shoulder Pads, Relevant Cords, Stoppers on Hanger | Net to Net |
| J180 | Gents Readymade Jackets and Overcoat Polyester-Wool Blended Fabrics (with/without lining) | 1 kg | 1 |  | Scoured Wool | 1.45 kg/kg content in export. |
|  |  |  | 2 |  | Polyester Staple Fibre | 1.211 kg/kg content in export. |
|  |  |  | 3 |  | Disperse Dyes (for dyed product only) | 2% of wt. of Polyester content in export. |
|  |  |  | 4 |  | Lining Fabric | 1.86 Sq. Mtrs. |
|  |  |  | 5 |  | Polyester Wadding | Net to net. |
| J181 | Gents Readymade Trousers/ Jackets/ Overcoat of 100% Raw Wool (With or without lining) | 1 kg | 1 |  | Scoured Wool | 1.45 kg/kg content in export. |
|  |  |  | 2 |  | Lining Fabrics (only for Jacket and Overcoat with lining) | 1.86 Sq. Mtrs. |
| J182 | Gents Trousers made of polyester viscose blended dyed processed and finished fabrics | 1 Kg. | 1 |  | Polyester staple Fibre | 1.211 kg/kg content in export |
|  |  |  | 2 |  | Viscose staple Fibre | 1.211 kg/kg content in export |
|  |  |  | 3 |  | Disperse dyes of 100% strength | 2% of wt. of polyester content in export. |
|  |  |  | 4 |  | Vat dyes of 100% strength | 1.5% of wt.of viscose content in export |
|  |  |  | 5 |  | Optical Whitening agent | 0.005 Kg. |
|  |  |  | 6 |  | Polyvinyl Alcohol | 0.01 Kg |
|  |  |  | 7 |  | Hydrogen Peroxide | 0.02 Kg |
|  |  |  | 8 |  | Silicon Emulsion | 0.02 Kg. |
| J183 | Girl's - Cotton Denim Full Sleeves Jackets with/without Embroidered/Patches etc. | 1 kg | 1 |  | Cotton Denim Fabric | 1.20 Sq.Mtr. |
| J184 | Grey Acrylic Knitwear (including General cut items) | 1 kg | 1 |  | Acrylic Yarn OR | 1.20 kg/kg content in export. |
|  |  |  |  |  | Acrylic Fibre | 1.30 kg/kg content in export. |
| J185 | Grey Placemat, Napkin, Table Cloth, Cushion Cover and Comfort Cover etc. | 1 kg | 1 |  | Grey Cotton Fabric | 1.01 kg/kg content in the export product |
| J186 | Grommet Band made from Nylon (Polyamide) base fabric | 1 kg | 1 |  | Relevant Nylon Fabric | 1.04 kg |
| J187 | Hospital Gowns made of Cotton | 1 No. | 1 |  | 100% Cotton Fabric | 2.3 Sq.Mtr. |
| J188 | Inner Lining for Jackets | 1 No. | 1 |  | Fabric to be used in the garment | 1.375sq.m |
| J189 | Inner Lining for vests | 1 No. | 1 |  | Fabric to be used in the garment | 0.825 sq.m |
| J190 | Jeans | 1 No. | 1 |  | Fabric to be used in the garment | 1.925 sq.m |
| J191 | Knitted Panels for Sweaters made from Acrylic Fibre (Dyed/Undyed) | 1 kg | 1 |  | Acrylic Fibre | 1.200 kg. |
|  |  |  | 2 |  | Acrylic Dyes of 100% Strength (for dyed product only) | 0.020 kg. |
| J192 | Knitted Panels for Sweaters made from Raw Wool (Dyed/Undyed) | 1 kg | 1 |  | Raw Wool | 1.350 kg. |
|  |  |  | 2 |  | Dyes (Acid/ Chrome) of 100% Strength (for dyed product only) | 0.020 kg. |
| J193 | Deleted |  |  |  |  |  |
| J194 | Deleted |  |  |  |  |  |
| J195 | Ladies Jackets with lining | 1 No. | 1 |  | Fabric to be used in the garment | 2.20 sq.m |
|  |  |  | 2 |  | Lining fabric to be used in the garment | 1.65 sq.m |
| J196 | Ladies Jackets without lining | 1 No. | 1 |  | Fabric to be used in the garment | 2.20 sq.m |
| J197 | Ladies Jump Suits | 1 No. | 1 |  | Fabric to be used in the garment | 3.51 sq.m |
| J198 | Ladies Long Coat | 1 No. | 1 |  | Fabric to be used in the garment | 4.5 sq.m |
|  |  |  | 2 |  | Lining fabric to be used in the garment | 4.05 sq.m |
| J199 | Ladies Night Wears | 1 No. | 1 |  | Fabric to be used in the garment | 2.695 sq.m |
| J200 | Deleted |  |  |  |  |  |
| J201 | Deleted |  |  |  |  |  |
| J202 | Deleted |  |  |  |  |  |
| J203 | Ladies Sleeves less jacket (Ladies Vests) made of Cotton Denim | 1 kg | 1 |  | Cotton Denim Fabrics | 1.15 kg/kg content in export. |
| J204 | Made-ups manufactured out of Polyester Cotton Blended Grey and Processed Fabrics | 1 kg | 1 |  | Polyester Staple Fibre | 1.115 kg/kg content in the export product. |
|  |  |  | 2 |  | Polyvinyl Alcohol | 1% on the weight of export product. |
| J205 | Made-ups of Grey Cotton Fabrics | 1 kg | 1 |  | Grey Cotton Fabrics OR | 1.02 kg/kg content in the export product |
|  |  |  | 1 |  | Raw Cotton | 1.195 kg/kg content in the export product |
| J206 | Nylon Socks | 1 kg | 1 |  | Nylon yarn (white) | 1.020 kg |
|  |  |  | 2 |  | Acid Dyes of 100% strength | 0.02 kg |
| J207 | Parka with lining | 1 No. | 1 |  | Fabric to be used in the garment | 4.20 sq.m |
|  |  |  | 2 |  | Lining fabric to be used in the garment | 3.78 sq.m |
| J208 | Parka with lining and Polyester Wadding | 1 No. | 1 |  | Fabric to be used in the garment | 4.20 sq.m |
|  |  |  | 2 |  | Lining fabric to be used in the garment | 3.78 sq.m |
|  |  |  | 3 |  | Polyester wadding | Net to net. |
| J209 | Polyester Fleece Knitted Garments with Hood | 1 No. | 1 |  | 100% Spun Polyester Knitted Fleece Fabric Dyed | 1.05 Sq.Mtr. per Sq.Meter content in export. |
|  |  |  | 2 |  | 100% Spun Polyester | 1.01 Sq.Mtr. per Sq.Meter content in export. |
| J210 | Polyester Wool Blended Knitwears | 1 kg | 1 |  | Scoured Wool | 1.35 kg/kg of Wool contents in the export product |
|  |  |  | 2 |  | Polyester Filament Yarn | 1.10 kg/kg of Polyester contents in export product. |
| J211 | Processed Quilted Cotton Made Ups other than Grey | 1 kg | 1 |  | Processd Cotton Fabrics | 1.02 kg/kg content |
|  |  |  | 2 |  | Raw Cotton | 1.02 kg/kg content in the export product |
| J212 | Raised Scarves Mufflers containing Polyester yarn and any other animal fibres like Mohair, Wool etc. | 1.0 Kg | 1 |  | Mohair Tops OR | 1.21 Kg./Kg. content in export. |
|  |  |  |  |  | Wool Tops OR | 1.21 Kg/Kg content in export. |
|  |  |  |  |  | Mohair Scoured/ Scoured Wool | 1.30 kg/kg content in export |
|  |  |  | 2 |  | Polyester Texturised yarn | 1.05 Kg./Kg. content in export. |
| J213 | Readymade garments Man's Suit Set made out of Nylon fabric/ Taffeta | 1 Set | 1 |  | Nylon Fabric | 5.0 sq. mtr. per set |
|  |  |  | 2 |  | Relevant Lining material | 3.5 Sq. mtrs. |
| J214 | Scoured, Bleached and Finished Placemat, Napkin, Table Cloth, Cushion Cover and Comfort Cover etc. | 1 kg | 1 |  | Grey Cotton Fabric | 1.03 kg/kg content in export product |
| J215 | Socks-Blended (made from cotton, Nylon/Polyester, Dyed Acrylic,Dyed Polyester Elastic Yarn) | 1.0 Kg | 1 |  | Crimped/ Texturised, single/double/ Nylon/Polyester Yarn. | 1.04 Kg./Kg. content in export. |
|  |  |  | 2 |  | Dyed high Bulk Acrylic yarn in Assorted Colours. | 1.04 Kg./Kg. content in export. |
|  |  |  | 3 |  | Elastic Yarn Rubber covered yarn. | 1.01 Kg./Kg. content in export. |
|  |  |  | 4 |  | Single/Double crimped Nylon yarn | 1.04 Kg./Kg. content in export. |
| J216 | Deleted |  |  |  |  |  |
| J217 | Deleted |  |  |  |  |  |
| J218 | Trousers made out of Polyester & Wool Blended Fabric | 1 kg | 1 |  | Scoured Wool | 1.45 kg/kg content in export product. |
|  |  |  | 2 |  | Polyester Tops OR | 1.29 Kg/Kg content in the export product. |
|  |  |  |  |  | Polyester Staple Fibres | 1.211 Kg/Kg content in the export product. |
|  |  |  | 3 |  | Disperse Dyes (100% Strength) | 2% of Polyester content in the export product. |
|  |  |  | 4 |  | Acid Dyes/Basic Dyes/Metal complex Dyes (100% Strength) | 2% of Wool content in the export product. |
|  |  |  | 5 |  | Softening Agent | 1% of weight of export product. |
| J219 | Velour Knitted readymade garments | 1 kg | 1 |  | Polyester Texturised yarn | 1.250 kg/kg content in export |
|  |  |  | 2 |  | Disperse Dyes of 100% Strength | 0.02 kg |
| J220 | Vest | 1 kg. | 1 |  | Relevant Knitted Fabric | 1.18 kgs. |
| J221 | Vest made of Nylon | 1 Pc. | 1 |  | Nylon Fabric with polyester wadding(quilted) | 1.05 sq.m. |
| J222 | Waist Coat made of Nylon | 1 Pc. | 1 |  | Nylon Fabric with Polyester wadding(quilted) | 1.16 sq.m. |
| J223 | Wind cheater with lining, Polyester wadding fitted with Elastic/ Cord at the bottom & cuff | 1 No. | 1 |  | Fabric to be used in the garment | 3.35 sq.m |
|  |  |  | 2 |  | Lining fabric to be used in the garment | 3.01 sq.m |
|  |  |  | 3 |  | Polyester wadding | Net content + 1% wastage. |
| J224 | Wool and Acrylic Blended Knitwears | 1 kg | 1 |  | Scoured Wool OR | 1.40 kg/kg content of wool in export. |
|  |  |  |  |  | Wool Tops | 1.30 kg/kg content of Wool in export |
|  |  |  | 2 |  | Acrylic Fibre | 1.300 kg/kg content of Acrylic in export. |
|  |  |  | 3 |  | PerchloroEthylene | 5% of net weight of wool in the export product |
| J225 | Woollen Knitwear | 1 kg | 1 |  | Scoured Wool OR Wool Tops OR Wool Yarn | 1.40 kg 1.30 kg 1.10 kg/kg content in the export product |
|  |  |  | 2 |  | Perchloroethylene | 0.050 kg |
| J226 | Woollen knitwears blended with Angora & Nylon | 1 kg | 1 |  | Woollen blended Yarn with Angora & Nylon | 1.138 kg. |
|  |  |  | 2 |  | Perchloroethylene | 5% of Net weight of wool in the export product |
| J227 | 100% Woollen Worsted Yarn (Dyed) | 1 Kg. | 1 |  | Wool Tops | 1.10 kg/kg content in export |
|  |  |  | 2 |  | Tinting Agent | 0.005 Kg. |
|  |  |  | 3 |  | Metal Complex Dyes of 100% strength | 1.5% of wt. of wool content in export. |
| J228 | 100% Woollen Worsted Yarn (Dyed/Grey) | 1 kg | 1 |  | Scoured Wool | 1.211 kg |
|  |  |  | 2 |  | Tinting Agent | 0.005 kg |
|  |  |  | 3 |  | Softening Agent | 0.010 kg |
|  |  |  | 4 |  | Metal Complex Dyes of 100% strength OR | 2% of weight of wool content in export. |
|  |  |  | 1 |  | Wool Tops | 1.10 kg |
|  |  |  | 2 |  | Tinting Agent | 0.005 kg |
|  |  |  | 3 |  | Metal Complex Dyes of 100% strength | 1.5% wt. of wool content in export. |
| Note.1 Import of dyes to be allowed only if export product is "dyed". |  |  |  |  |  |  |
| J229 | 100% Woollen Worsted Yarn (Grey) | 1 kg | 1 |  | Scoured Wool | 1.211 kg. |
|  |  |  | 2 |  | Tinting Agent | 0.005 kg |
|  |  |  | 3 |  | Softening Agent | 0.010 kg |
| J230 | 100% Woollen Worsted Yarn(Grey) | 1 Kg. | 1 |  | Wool Tops | 1.10 Kg/kg content in export |
|  |  |  | 2 |  | Tinting Agent | 0.005 Kg. |
| J231 | Blanketing Cloth | 1 kg. | 1 |  | Synthetic Waste | 1.06 kg./kg. content in the export product |
|  |  |  | 2 |  | Wool Waste | 1.25 kg./kg. content in the export product |
|  |  |  | 3 |  | Woollen Rags/synthetic Rags | 1.55 kg./kg.content in the export product |
|  |  |  | 4 |  | Scoured Wool | 1.25 kg./kg. content in the export product |
| J232 | Blankets containing wool and polyester | 1 Kg | 1 |  | Scoured wool | 1.32 Kg/Kg content in export. |
|  |  |  | 2 |  | Polyester Filament Yarn | 1.05 Kg./Kg. content in export. |
| J233 | Blankets/Blazer/Vellour/ Overcoat/Jacket Cloth made of Synthetic Waste/ Woollen Waste/ Woollen Rags/ Scoured Wool/Synthetic Rags | 1 kg | 1 |  | Synthetic Waste | 1.060 kg/kg content in export |
|  |  |  | 2 |  | Wool Waste | 1.310 kg/kg content in export |
|  |  |  | 3 |  | Woollen Rags/ Synthetic Rags | 1.670 kg/kg content in export |
|  |  |  | 4 |  | Scoured wool | 1.310 kg/kg content in export |
|  |  |  | 5 |  | Metal Complex Dyes (For wool only) | 0.02 kg. |
|  |  |  | 6 |  | Acrylic Dyes (For acrylic only) | 0.02 kg. |
|  |  |  | 7 |  | Relevant Pigments/Naphthol/Direct/Reactive Dyes (For viscose only) | 0.02 kg. |
|  |  |  | 8 |  | Woollen/Synthetic Clipping | 1.30 kg/kg content in the export product |
|  |  |  | 9 |  | Pulled or Garnetted Fibres made out of Woollen and/or Synthetic Rags/ Clippings/ Hard waste | 1.11 kg/kg content in the export product |
| J234 | Dyed Wool Fabrics | 1 kg | 1 |  | Scoured Wool OR Wool Tops | 1.308 kg/kg of Wool content in the export product. 1.198 kg/kg content of wool in export |
|  |  |  | 2 |  | Acid Dyes or Metal Complex Dyes | 2% of Wool content in the export product. |
|  |  |  | 3 |  | Tinting Agent | 0.5% of the weight of export product. |
|  |  |  | 4 |  | Softening Agent - and/or Moth Proofing Agent | 0.5% of the Wool content in the export product. |
|  |  |  | 5 |  | Elastomer Yarn | Net to net |
| Note: In case, the export product is Teflon Treated, an additional item - Oleophobol-S/ Hydrophobol-X may be allowed @0.04 kg/kg of the export product. |  |  |  |  |  |  |
| J235 | Fabrics made of 100% wool | 1 kg | 1 |  | Scoured Wool | 1.308 kg |
| J236 | Knitted Woollen Mufflars (Dyed/ Undyed) | 1 kg | 1 |  | Woollen Yarn not Exceeding 32 BWS | 1.100 kg. |
|  |  |  | 2 |  | Dyes (Acid/ Chrome) of 100% strength (only for dyed export product) | 0.020 kg. |
| J237 | Knitted Woollen Shawls (Dyed/ Undyed) | 1 kg | 1 |  | Woollen/ worsted yarn not exceeding 32 BWS | 1.100 kg. |
|  |  |  | 2 |  | Dyes (Acid/ chrome) of 100% strength (only for dyed export product) | 0.020 kg |
| J238 | Mohair and Polyester blended knitwears | 1 Kg | 1 |  | Mohair top OR Mohair Scoured | 1.300 kg/kg content in export 1.350 kg/kg content in export |
|  |  |  | 2 |  | Polyester staple fibre | 1.125 kg/kg content in export |
|  |  |  | 3 |  | Perchloro ethylene | 5% of export weight of wool and woolen blended items |
| J239 | Mohair Polyester Mixed Blankets Raised | 1 kg | 1 |  | Mohair Tops | 1.26 kg./kg. content in export product. |
|  |  |  | 2 |  | Polyester Texturised Yarn | 1.05 kg/kg content in export product. |
| J240 | Mohair, Acrylic, Polyester Nylon and wool Dyed blended knitwears | 1 kg. | 1 |  | Mohair top OR Mohair Scoured | 1.30 kg/ kg content in the export product 1.35 kg/ kg. content in the export product |
|  |  |  | 2 |  | Acrylic fibre | 1.30 kg/ kg content in the export product |
|  |  |  | 3 |  | Polyester Filament Yarn | 1.10 kg/kg content in the export product |
|  |  |  | 4 |  | Nylon Fibre | 1.115 kg/ kg content in the export product |
|  |  |  | 5 |  | Scoured Wool OR Wool Top | 1.40 kg/kg content in the export product. 1.30 kg/kg content in the export product. |
|  |  |  | 6 |  | Perchloro ethylene | 5% of weight of Mohair in the export product |
|  |  |  | 7 |  | Acrylic Dyes of 100% strength (Cationic Dyes) | 2% of weight of Acrylic content in the export product. |
|  |  |  | 8 |  | Disperse Dyes of 100% Strength | 2% of weight of Polyester content in the export product. |
|  |  |  | 9 |  | Acid Dyes of 100% Strength | 2% of weight of Nylon content in the exportproduct. |
|  |  |  | 10 |  | Acid Dyes of 100% Strength | 1.5% of weight of Wool content in the export product. |
| J241 | Pressed Wool Felts/Compressed Wool Felts/Wool Felts made of 100% Wool/Carbonized Wool/Wool Waste and articles thereof | 1 Kg | 1 |  | Scoured Wool | 1.13 Kg./Kg. content of wool/scoured wool/wool waste in export. |
| J242 | Pressed Wool Felts/Compressed Wool Felts/Wool Felts made of 100% Carbonized Wool/ Wool Waste and articles thereof | 1 kg | 1 |  | Wool Waste OR a)Carbonised Wool b) Carbonised Wool Waste | 1.08 Kg./Kg content of wool in export |
| J243 | Shawls/Scarves/Mufflers & Made-ups made of 100% wool | 1 kg | 1 |  | Scoured wool OR Wool Top OR Wool Yarn | 1.308 kg/kg content in the export product 1.198 kg/kg content in the export product 1.10 kg/kg content in the export product |
| J244 | Wool Blazer/Woollen Blended Fabrics containing Wool & Polyester/ Viscose. | 1 kg | 1 |  | Scoured wool | 1.308 kg/kg content in export |
|  |  |  | 2 |  | Polyester Fibre OR Polyester Tow | 1.211 kg/kg content in export 1.17 kg/kg of Polyester content in export |
|  |  |  | 3 |  | Viscose Fibre | 1.211 kg/kg content in export |
| J245 | Wool tops Dyed/ Undyed | 1 kg | 1 |  | Scoured wool | 1.110 kg |
|  |  |  | 2 |  | Metal Complex Dyes of 100% Strength (only for dyed export product) | 1.5% of weight of wool in export. |
| J246 | Woollen Blended Knitwears | 1 kg | 1 |  | Woollen Yarn (not exceeding 32 BWS) | 1.10 kg/kg of Wool content in the export product. |
| J247 | Woollen Blended Shawls/ Scarves/ Mufflers containing Wool & Polyester/ Viscose. | 1 kg | 1 |  | Scoured wool | 1.308 kg/kg content in export |
|  |  |  | 2 |  | Polyester Fibre | 1.211 kg/kg content in export |
|  |  |  | 3 |  | Viscose Fibre | 1.211 kg/kg content in export |
| J248 | Woollen Blended Yarn containing Polyester/Wool/Acrylic/Viscose/Nylon (Grey/Dyed) | 1 kg | 1 |  | Polyester Fibre/ Polyester Staple Fibre OR Polyester Tow | 1.13 kg/kg content in the export product. 1.08 kg/kg content in the export product |
|  |  |  | 2 |  | Scoured Wool OR Wool Tops | 1.211 kg/kg content in the export product. 1.10 kg/kg content in the export product |
|  |  |  | 3 |  | Acrylic Fibre OR Acrylic Tow | 1.13 kg/kg content in the export product 1.05 kg/kg content in the export product |
|  |  |  | 4 |  | Viscose Fibre/ Viscose Staple Fibre | 1.115 kg/kg content in the export product. |
|  |  |  | 5 |  | Tinting Agent | 0.005 kg |
|  |  |  | 6 |  | Softening Agent | 0.010 kg |
|  |  |  | 7 |  | Disperse Dyes of 100% Strength | 2% of wt. of Polyester content in export product. |
|  |  |  | 8 |  | Acrylic Dyes of 100% Strength | 2% of wt. of Acrylic content in export product. |
|  |  |  | 9 |  | Metal Complex Dyes of 100% Strength | 2% of wt. of Wool content in export product. |
|  |  |  | 10 |  | Pigment/ Napthol/ Direct/ Reactive Dyes of 100% strength | 2% of wt. of Viscose content in export product. |
|  |  |  | 11 |  | Nylon Fibre OR Nylon Tops | 1.11 kg/kg content in the export product 1.08 kg/kg content in the export product |
| J249 | Woollen Shoddy Yarn | 1 kg. | 1 |  | Woollen Rags | 1.4 kg./kg. content in the export product |
|  |  |  | 2 |  | Wool Waste | 1.15 kg./kg content in the export product |
|  |  |  | 3 |  | Woolen Clipping | 1.15 kg/kg content in the export product |
|  |  |  | 4 |  | Pulled or Garnetted Fibres made out of Woollen and/or Synthetic Rags/ Clippings/ Hard waste | 1.05 kg/kg content in the export product |
| J250 | Woollen Yarn | 1 kg. | 1 |  | Scoured Wool OR Carbonised Wool | 1.15 kg./kg. content in the export product. 1.12 kg/kg content in the export product |
| J251 | Worsted Spun Yarn(Dyed) containing Polyester and wool | 1 Kg. | 1 |  | Polyester Fibre OR | 1.13 kg/kg content in export |
|  |  |  | 1 |  | Polyester Tow | 1.08 kg/kg content in export. |
|  |  |  | 2 |  | Wool Tops | 1.10 kg/kg content in export |
|  |  |  | 3 |  | Tinting Agent | 0.005 Kg |
|  |  |  | 4 |  | Disperse Dyes of 100% Strength | 2% of wt. of polyester content in export |
|  |  |  | 5 |  | Metal Complex Dyes of 100% Strength | 1.5% of weight of wool content in export |
| J252 | Men's /Ladies Riding Breeches of Woven/ Knitted Fabric with Full Seat of Artificial Leather Cloth | 1 No. | 1 |  | Relevant type of Fabric | 2.12 Sq.Mtr. |
|  |  |  | 2 |  | Artificial Leather cloth | 1.05 kg/kg content in the export product |
|  |  |  | 3 |  | Hook Fastners 1 set of 4 pcs. | Net to net |
|  |  |  | 4 |  | Zips of relevant size and specification | Net to net |
| J253 | Woven Muffler containing Mohair Yarn and Polyester Filament Yarn | 1 Kg. | 1 |  | Mohair Top | 1.18 kg/kg content in the export product. |
|  |  |  | 2 |  | Polyester Texturised Yarn | 1.05 kg/kg content in the export product. |
| J254 | Dyed/Printed Polyester-Viscose Fabric Blended | 1 Kg. | 1 |  | Polyester-Viscose Blended Fabric (Grey) | 1.02 kg/kg content in the export product. |
|  |  |  | 2 |  | Disperse Dye (100% Strength) | 0.02 kg/kg of Polyester material content in the export product. |
| J255 | Dyed/Dyed and Printed Fabrics/ Sarees | 1 Kg | 1 |  | Grey Polyester Fabric | 1.03 kg/kg content in export. |
|  |  |  | 2 |  | Relevant Dyes (100% Strength) | 2% of weight of Polyester content in the export product. |
| J256 | Polyester Partially Oriented Yarn | 1 Kg. | 1 |  | Polyester Chips (Textile Grade) | 1.05 kg |
|  |  |  | 2 |  | Spin Finish Oil | 0.010 kg |
|  |  |  | 3 |  | Dowtherm (Therminol) | 0.005 kg |
|  |  |  | 4 |  | LDO/ Furnace Oil | 0.182 Ltr. /kg. of Polyester Partially Oriented yarn |
| J257 | Boys/Girls Work Shorts Size 2 to 6 | 1 Pc. | 1 |  | Fabrics to be used in the garment | 1.058 Sq. Mtrs. |
| J258 | Boys/Girls work Shorts Size 7 to 14 | 1 Pc. | 1 |  | Fabrics to be used in the garment | 1.558 Sq. Mtrs. |
| J259 | Boys/Girls Adventure Shorts Size 2 to 6 | 1 Pc. | 1 |  | Fabrics to be used in the garment | 1.045 Sq. Mtrs. |
| J260 | Boys/Girls Adventure Shorts size 7 to 14 | 1 Pc. | 1 |  | Fabrics to be used in the garment | 1.375 Sq. Mtrs. |
| J261 | Boys/Girls Work Trouser Size 2 to 6 | 1 Pc. | 1 |  | Fabrics to be used in the garment | 1.572 Sq. Mtrs. |
| J262 | Boys/Girls work Trouser size 7 to 14 | 1 Pc. | 1 |  | Fabrics to be used in the garment | 2.042 Sq. Mtrs. |
| J263 | Boys/Girls Adventure Trouser Size 2 to 6 | 1 Pc. | 1 |  | Fabrics to be used in the garment | 1.459 Sq. Mtrs. |
| J264 | Boys/Girls Adventure Trouser Size 7 to 14 | 1 Pc. | 1 |  | Fabrics to be used in the garment | 1.782 Sq. Mtrs. |
| J265 | Pants for Infants | 1 No. | 1 |  | Fabrics to be used in the garment | 0.79 Sq. Mtr. |
| J266 | Boiler Suit, made out of Polyester Cotton Dyed Cloth | 1 Kg. | 1 |  | Polyester Staple Fibre | 1.211 kg/kg of Polyester content in the export product. |
|  |  |  | 2 |  | Disperse Dyes of 100% strength | 2% on the wt. of Polyester content in the export product. |
|  |  |  | 3 |  | Vat Dyes of 100% strength | 1.5% on the wt. of Cotton content in the export product. |
| J267 | Men's Long Sleeve Quilted Shirt | 1 Pc. | 1 |  | Quilted Fabric OR Fabric to be used in the garment (outer fabric) | 2.45 Sq. Mtrs. 2.45 sq.mtrs. |
|  |  |  | 2 |  | Relevant inner fabric | 2.45 sq.mtr. |
|  |  |  | 3 |  | Polyester Staple Fibre | 140 gms |
| J268 | Trousers made of Leather & Knitted Fabric | 1 No. | 1 |  | Knitted Fabric width 150 Cms(60") for Size:- |  |
|  |  |  |  |  | Extra Large | 1.05 Sq.mtr. |
|  |  |  |  |  | Large | 0.99 Sq.mtr. |
|  |  |  |  |  | Medium | 0.95 Sq.mtr. |
|  |  |  |  |  | Small | 0.90 Sq.mtr. |
| J269 | Polyester Filament Yarn | 1 Kg. | 1 |  | Polyester Chips (Textile Grade) | 1.05 kg |
|  |  |  | 2 |  | Spin Finish Oil | 0.0101 kg |
|  |  |  | 3 |  | Dowtherm (Heat transfer oil) | 0.0047 kg |
|  |  |  | 4 |  | LDO/ Furnace Oil | 0.182 Ltr. /kg. of Polyester Filament yarn |
| J270 | Ladies Shirt (Full Sleeve) | 1 Pc. | 1 |  | Fabric to be used in the Garment | 2.75 Sq. Mtrs. |
| J271 | Ladies Shirt (Half Sleeve) | 1 Pc. | 1 |  | Fabric to be used in the Garment | 2.20 Sq. Mtrs. |
| J272 | Men's Shirt (Full Sleeve) | 1 Pc. | 1 |  | Fabric to be used in the Garment | 2.90 Sq. Mtrs. |
| J273 | Children Shirt (Full Sleeve) | 1 Pc. | 1 |  | Fabric to be used in the Garment | 2.25 Sq. Mtrs. |
| J274 | Children Shirt (Half Sleeve) | 1 Pc. | 1 |  | Fabric to be used in the Garment | 1.90 Sq. Mtrs. |
| J275 | Men's/Ladies Trouser | 1 Pc. | 1 |  | Fabric to be used in the Garment | 2.25 Sq. Mtrs. |
| J276 | Ladies Trouser with Lining | 1 Pc. | 1 |  | Fabric to be used in the Garment | 2.25 Sq. Mtrs. 1.10 Sq. Mtr. (For Lining) |
| J277 | Men's/Ladies T-Shirt (Full Sleeve) | 1 Pc. | 1 |  | Fabric to be used in the Garment | 2.60 Sq. Mtr. |
| J278 | Men's/Ladies T-Shirt (Short Sleeve) | 1 Pc. | 1 |  | Fabric to be used in the Garment | 2.20 Sq. Mtrs. |
| J279 | Children T-Shirt (Large Size) | 1 Pc. | 1 |  | Fabric to be used in the Garment | 2.10 Sq. Mtrs. |
| J280 | Gents/Ladies Sweat Shirt (Full Sleeve) | 1 Pc. | 1 |  | Fabric to be used in the Garment | 2.90 Sq. Mtrs. |
| J281 | Children Sweat Shirt (Full Sleeve) | 1 Pc. | 1 |  | Fabric to be used in the Garment | 2.30 Sq. Mtrs. |
| J282 | Men's Pyjama Set (Please see Note.1 below) | 1 Set | 1 |  | Fabric to be used in the Garment | 4.80 Sq. Mtrs. |
| J283 | Ladies Pyjama Set (Please see Note.1 below) | 1 Set | 1 |  | Fabric to be used in the Garment | 4.50 Sq. Mtrs. |
| J284 | Children Pyjama Set (Please see Note.1 below) | 1 Set | 1 |  | Fabric to be used in the Garment | 3.85 Sq. Mtrs. |
| J285 | Men's Night Dress (Please see Note.2 below) | 1 Pc. | 1 |  | Fabric to be used in the Garment | 4.30 Sq. Mtrs. |
| J286 | Ladies Night Dress (Please see Note.2 below) | 1 Pc. | 1 |  | Fabric to be used in the Garment | 4.00 Sq. Mtrs. |
| J287 | Children Night Dress (Please see Note.2 below) | 1 Pc. | 1 |  | Fabric to be used in the Garment | 3.45 Sq. Mtrs. |
| J288 | Ladies Dress (Please see Note.3 below) | 1 Pc. | 1 |  | Fabric to be used in the Garment | 4.5 Sq. Mtr. & 2.00 Sq. Mtr. for Lining |
| J289 | Ladies Skirts | 1 Pc. | 1 |  | Fabric to be used in the Garment | 3.15 Sq. Mtrs. |
| J290 | Ladies Blouse | 1 Pc. | 1 |  | Fabric to be used in the Garment | 2.45 Sq. Mtrs. |
| J291 | Girls Blouse | 1 Pc. | 1 |  | Fabric to be used in the Garment | 1.85 Sq. Mtrs. |
| J292 | Men's/Ladies Shorts | 1 Pc. | 1 |  | Fabric to be used in the Garment | 1.60 Sq. Mtrs. |
| J293 | Men's Brief | 1 Pc. | 1 |  | Fabric to be used in the Garment | 0.70 Sq. Mtrs. |
| J294 | Ladies Panties | 1 Pc. | 1 |  | Fabric to be used in the Garment | 0.60 Sq. Mtrs. |
| J295 | Co-ordinate Set (Comprising of Full Sleeve Top and Trouser) | 1 Pc. | 1 |  | Fabric to be used in the Garment | 4.80 Sq. Mtrs. |
| J296 | Co-ordinate Set (Comprising of Half Sleeve T-Shirt & Short) | 1 Pc. | 1 |  | Fabric to be used in the Garment | 3.80 Sq. Mtrs. |
| Note: 1) PYJAMAS: Consisting of two garments, namely, a garment intended to cover the upper part of the body, generally a jacket type garment or pullover or similar type of garment and a garment consisting of a trouser or short of simple cut with no opening or with an opening at the front. 2) NIGHT DRESS: These garments are made from lighter fabric, vary in length from less than knee length to ankle length. Such night dresses may be eleborately styled with decoration e.g.lace, ribbons, flounces, bows, embroidery, pipings, gathers etc. When made from comparatively heavier fabrics which may be brushed and/or printed, they are more simply styled and may have some decoration. 3) LADIES DRESS: The product one piece of garment for the female covering the top of the body and extending to some where from the mid-thigh to feet. It is appropriate to wear without other garments and its lower end encloses both the legs in a single "tube" (rather than two, as trousers do). |  |  |  |  |  |  |
| J297 | Twisted Silk Yarn | 1 kg | 1 |  | Relevant Raw Silk | 1.01 kg/kg content in the export product. |
| J298 | Furnishing Fabric made out of Dyed Polyester Filament Yarn & Dyed Cotton Yarn | 1 kg | 1 |  | Grey Polyester Filament Yarn | 1.10 kg/kg content in the export product. |
|  |  |  | 2 |  | Cotton Yarn Undyed | 1.10 kg/kg content in the export product. |
|  |  |  | 3 |  | Disperse Dyes of 100% Strength | Upto 2% of weight of Polyester Filament Yarn content in the export product. |
|  |  |  | 4 |  | Reactive Dyes of 100% Strength | Upto 1.5% of weight of cotton yarn in the export product. |
| J299 | Cotton Woven Interlining | 1 kg | 1 |  | Cotton Processed Fabrics | 1.01 kg/kg content in export. |
|  |  |  | 2 |  | LDPE/HDPE Powder | 15% of weight of Cotton Fabric in export. |
| Note: HDPE Powder shall be allowed, if the product to be exported is made of Microdot Application Method |  |  |  |  |  |  |
| J300 | Athletic & Swim Shorts with short legs and slits in each side | 1 No. | 1 |  | Fabric to be used in the Garment | 0.90 Sq. mtrs. |
| J301 | Industrial Workwear made out of: Polyester Cotton Blended High Visibility Dyed Fabric with Reflective Band/Polyester High Visibility Dyed Fabric with Reflective Band. a) Boiler Suit b) Jacket c) Pant | 1 Pc. |  |  |  |  |
|  |  |  |  | a) | Relevant Fabric | 4.05 Sq. Mtr. |
|  |  |  |  | b) | Relevant Reflective Band | 5 cm or 6 cm width X length upto 7 mtr. |
|  |  |  |  | a) | Relevant Fabric | 2.48 Sq. Mtr. |
|  |  |  |  | b) | Relevant Reflective Band | 5 cm or 6 cm width X length upto 4.6 mtr |
|  |  |  |  | a) | Relevant Fabric | 1.687 Sq. Mtr. |
|  |  |  |  | b) | Relevant Reflective Band | 5 cm or 6 cm width X length upto 2.4 mtr |
|  | d) Bib Pant |  |  | a) | Relevant Fabric | 2.30 sq mtr. |
|  |  |  |  | b) | Relevant Reflective Band | 5 cm or 6 cm width X length upto 3.6 mtr |
|  | e) Gilet Waist Coat |  |  | a) | Relevant Fabric | 1.16 Sq. Mtr. |
|  |  |  |  | b) | Relevant Reflective Band | 5 cm or 6 cm width X length upto 2.85 mtr |
|  | f) Short Coat |  |  | a) | Relevant Fabric | 2.70 Sq. Mtr. |
|  |  |  |  | b) | Relevant Reflective Band | 5 cm or 6 cm width X length upto 4.6 mtr |
| Note: The import of reflective band is subject to actual user condition only. |  |  |  |  |  |  |
| J302 | Children (Infants) Cotton Denim Full Sleeve Jackets (Embroidered) | 1 No. | 1 |  | Cotton Denim Fabric | 1.20 Sq. Mtr. |
| J303 | Dyed Upholstry/Furnishing Fabrics made out of Polyester Filament Yarn/Viscose Spun Yarn/Cotton Yarn | 1 kg | 1 |  | Polyester Filament Yarn | 1.10 kg/kg content in the product |
|  |  |  | 2 |  | Viscose Spun Yarn | 1.10 kg/kg content in the export product |
|  |  |  | 3 |  | Cotton Yarn | 1.10 kg/kg in the export product |
|  |  |  | 4 |  | Disperse Dyes of 100% Strength | 2% of weight of polyester content in the export product |
|  |  |  | 5 |  | Vat Dyes | 1.5% of weight of Viscose content in the export product |
|  |  |  | 6 |  | Reactive Dyes | 1.5% of weight of Cotton content in the export product |
| J304 | Silk Wool Fabrics | 1 kg | 1 |  | Spun Silk Yarn | 1.050 kg/kg content in the export product. |
|  |  |  | 2 |  | Scoured Wool | 1.300 kg/kg content in the export product . |
| J305 | Deleted |  |  |  |  |  |
| J306 | Polyester Woven Labels | 1 kg | 1 |  | Texturised Polyester Filament Yarn | 1.05 kg/kg content in the export product. |
| J307 | Polyester/Viscose/Cotton Blended Grey/Dyed Yarn | 1 kg | 1 |  | Polyester Staple Fibre Grey | 1.05 kg/kg content in the export product. |
|  |  |  | 2 |  | Viscose Staple Fibre Grey | 1.05 kg/kg content in the export product. |
|  |  |  | 3 |  | Disperse Dyes of 100% Strength | 0.020 kg/kg Polyester content in the export product. |
|  |  |  | 4 |  | Vat Dyes of 100% Strength | 0.015 kg/kg Viscose content in the export product. |
| Note: In case import item is Polyester Staple Fibre Dyed and Viscose StapleFibre Dyed then import of Disperse Dyes shall not be permissible. |  |  |  |  |  |  |
| J308 | Cotton yarn dyed fabrics/made-ups with/without elaston/lycra/spandex yarn | 1 kg | 1 |  | Cotton Yarn | 1.10 kg |
|  |  |  | 2 |  | Polyvinyl Alcohol (PVA) Sizing Materials | 0.01 kg |
|  |  |  | 3 |  | Sodium Hydro Sulphite | 0.02 kg |
|  |  |  | 4 |  | Vat/ Reactive/ Sulphur/ Pigment Dyes (100% Strength) | 0.015 kg/kg of cotton content in export product. |
|  |  |  | 5 |  | Elaston/lycra/spandex yarn OR Stretch yarn (mixed yarn containing 95% cotton and 5% of elaston/lycra/spandex yarn) | 1.05 kg/kg of materials content in the export product 1.05 kg/kg of materials content in the export product |
|  |  |  | 6 |  | Fuel for power generation Naphtha OR Furnace Oil | 5.50 kg 4.10 litres |
| Note: 1. Naphtha and Furnace Oil shall be allowed only under AU condition. Note: 2. The above norms for fuel are applicable only to composite units having spinning, weaving and processing activities. Also the said units should have a captive power plant using "Naphtha/Furnace Oil" as fuel. |  |  |  |  |  |  |
| J309 | Braided Elastic Tape made out of Polyester Filament Yarn and Heat Resistance Rubber Thread in assorted sizes | 1 kg | 1 |  | Polyester Filament Yarn(150 Denier) (+/-10%) | 1.02 kg/kg content in the export product. |
|  |  |  | 2 |  | Heat Resistance Rubber Thread(Guage 36 to 44) | 1.02 kg/kg content in the export product. |
| J310 | Ladies Sleevless Jackets (45% Polyester & 55% Nylon) with/ without lining & Polyester Wadding | 1 No. | 1 |  | 2 Tone Taffeta Azo Free Fabric containing 45% Polyester and 55% Nylon | 1.95 Sq.Mtr. |
|  |  |  | 2 |  | 190T 100% Nylon Taffeta Poly/ Azo Freez Lining | Net content + 5% wastage. |
| J311 | Flax Madeups | 1 kg | 1 |  | Flax Yarn | 1.05 kg/kg content in the export product. |
| J312 | Wool/Silk blended Shawls/Scarves/Mufflers and made-ups | 1 kg. | 1 |  | a)Scoured Wool OR b)Wool Top OR c)Wool Yarn | 1.31 Kg/kg content in the export product 1.21 Kg/kg content in the export product 1.10 kg/kg content in the export product |
|  |  |  | 2 |  | Spun Silk Yarn | 1.05 kg/kg content in the export product |
| J313 | Girls frock | 1 no. | 1 |  | Relevant fabric to be used in the garment | 2.50 sq.mtr. |
| J314 | Made-ups made from polyester cotton blended fabric | 1 No. | 1 |  | Polyester/cotton blended fabric of relevant GSM | 1.01 sq.mtr. per sq.mtr. content in the export product |
| J315 | i) Dyed man-made fabric ii) Dyed and printed man-made fabric/printed man made fabric | 1 sq.metre. 1 sq.mtr. | 1 |  | Relevant Man made grey fabric | 1.01 sq.mtr. per sq.mtr. content in the export product |
|  |  |  | 2 |  | Relevant Man-made dyed and/or Bleached fabric OR Relevant man made grey fabric | 1.01 Sq. Mtr. per sq.mtr. content in the export product 1.02 sq.mtr. per sq.mtr. content in the export product |
| J316 | Made-ups made from Man-made fabrics | 1 Sq. Metre | 1 |  | Relevant Processed Man-made fabrics | 1.01 Sq. metre per sq.mtr. content in the export product |
| J317 | Subaiya/Khanga made from 100% Viscose Fabrics with or without Fringes with or without Lurex | 1 Sq. Metre | 1 |  | 100% Viscose Fabric | 1.01 Sq. Metre per sq.mtr. content in the export product |
| J318 | Scarves/ Stoles/ Pareo/ Handkerchief | 1 Sq. Metre | 1 |  | Fabric to be used in the export product | 1.01 Sq.Metre per sq.mtr. content in the export product |
| J319 | Polyester Cotton Blended Grey Fabrics | 1.0 Kg. | 1 |  | Polyester Cotton Yarn of relevant blend | 1.05 kg/kg content in the export product |
| J320 | Polyester/Cotton Blended Dyed/ Printed operation Gowns or operation suits | 1.0 Kg. | 1 |  | Polyester staple fibre | 1.18 kg/kg content in the export product |
|  |  |  | 2 |  | Polyvinyl Alcohol | 0.01 kg/kg content in the export product |
|  |  |  | 3 |  | Raw Cotton | 1.20 kg/kg content in the export product |
|  |  |  | 4 |  | Disperse Dyes | 0.02 kg. |
|  |  |  | 5 |  | Vat Dyes | 0.02 kg. |
| J321 | Polyester/ Cotton Blended Hospital Garments such as operation gowns suits, jackets, trouser, aprons | 1.0 Kg. | 1 |  | Polyester staple fibre | 1.18 kg/kg content in the export product |
|  |  |  | 2 |  | Polyvinyl Alcohol | 0.01 kg/kg content in the export product |
|  |  |  | 3 |  | Raw Cotton | 1.20 kg/kg content in the export product |
| J322 | Acrylic Top | 1.0 Kg. | 1 |  | Acrylic Fibre | 1.01 kg/kg content in the export product |
| J323 | Dyed and/or printed synthetic blanket made out of Acrylic Spun Yarn (under cotton system) and Polyester Yarn containing 79% Acrylic Yarn, 13% Polyester Yarn and 8% Polyester Cotton Yarn | 1 Kg. | 1 |  | Acrylic Spun Yarn | 1.09 kg/kg content in the export product |
|  |  |  | 2 |  | Polyester Filament Yarn | 1.05 kg/kg content in the export product |
|  |  |  | 3 |  | Polyester Cotton Blended Yarn | 1.05 kg/kg content in the export product |
|  |  |  | 4 |  | Lylex Cloth | On net to net basis |
|  |  |  | 5 |  | Cationic Dyes of 100% Strength | 2% of the net content of the acrylic spun yarn in the export product |
|  |  |  | 6 |  | Zipper Poly bag | On net to net basis |
|  |  |  | 7 |  | Corrugated box | On net to net basis |
| J324 | Viscose/ Silk Blended Spun Dyed Yarn containing Viscose staple fibre dyed and silk top | 1 Kg. | 1 |  | Viscose Staple Fibre dyed | 1.05 kg/kg content in the export product |
|  |  |  | 2 |  | Silk Top | 1.05 kg/kg content in the export product |
| J325 | Cotton Printed Fabrics | 1 Sq. mtr. | 1 |  | Relevant Cotton processed fabrics | 1.01 sq.mtr. |
| J326 | Polyester Jacketted fire hose without lining | 1 Kg. | 1 |  | Polyester Staple fibre | 1.10 kg/kg content in the export product |
|  |  |  | 2 |  | Polyester filament yarn | 1.04 kg/kg content in the export product |
| J327 | Dyed / dyed & printed polyester fabrics/ sarees (by weight reduction process) | 1 No. | 1 |  | Grey Polyester fabrics | 1.13 kg/kg content in the export product |
|  |  |  | 2 |  | Relevant dyes of 100% strength | 2% weight of polyester content in the export product |
| Note: The exporter at the time of exports will supply a sample of grey and processed fabric to the Customs Authority supported by a Certificate issued from the testing Laboratories of the Textile Committee/ Man made Textiles Research Association (MANTRA)/ silk & Art silk Mills Research Association (SASMIRA), Bombay / Bombay Textiles Research Association (BTRA), Bombay / Southern India Textiles Research Association (SITRA), Coimbatore / Ahmedabad Textiles Industries Research Association (ATIRA), Ahmedabad or any other institution recognised by the Government of India for this purpose |  |  |  |  |  |  |
| J328 | Tie made from relevant man-made processed fabrics | 1 Kg. | 1 |  | Relevant man made processed fabrics | 1.05 kg/kg content in the export product |
| J329 | Lace made out of polyester filament yarn | 1 kg. | 1 |  | Polyester filament yarn | 1.10 kg/kg content in the export product |
|  |  |  | 2 |  | Disperse dyes of 100% strength | 2% wt. of polyester content in the export product |
| Note: If the export item is not dyed then the import item at sl.no.2 above shall not be allowed |  |  |  |  |  |  |
| J330 | Double layer cotton woven interlining/cotton woven interlining | 1 Kg. | 1 |  | Cotton fabric grey | 1.02 kg/kg content in the export product |
| J331 | Ladies Midi | 1 No. | 1 |  | Fabric to be used in the garment | 4 Sq. Metres. |
| J332 | Ladies Wrap Skirt | 1 No. | 1 |  | Fabrics to be used in the garment | 1.5 sq.mtr. |
| J333 | Ladies Chemise | 1 No. | 1 |  | Fabrics to be used in the garment | 1.50 sq.mtr. |
| J334 | Ladies Nightwear Set | 1 Set | 1 |  | Fabric to be used | 2.80 sq.mtr. |
| J335 | Boy's Long Sleeve Quilted shirt | 1 pc. | 1 |  | Fabric to be used (outer fabric) | 2 sq.mtr. |
|  |  |  | 2 |  | Relevant inner fabric | 2 sq.mtr. |
|  |  |  | 3 |  | Polyester staple fibre | 120 gms. |
| J336 | Ladies V-neck sleeveless beachwear blouse | 1 No. | 1 |  | Fabric to be used | 1.90 sq.mtr. |
| J337 | 100% Polyester Carbonised Printed Fabric | 1 sq.mtr. | 1 |  | 65% Polyester / 35% Cotton Blended fabric (Dyed/Printed) | 1.01 sq.mtr. |
| J338 | Children Vest | 1 pc. | 1 |  | Fabric to be used | 0.80 sq.mtr. |
| J339 | Ladies Singlet | 1 pc. | 1 |  | Fabric to be used | 0.60 sq.mtr. |
| J340 | Ladies Tube Top | 1 pc. | 1 |  | Fabric to be used | 0.50 sq.mtr. |
| J341 | Poly/Cotton knitted gloves - seamless with/without PVC dots (one side) | 1 pair | 1 |  | Poly/cotton yarn | 1.05 kg/kg content in the export product |
|  |  |  | 2 |  | PVC compound (Palstisol) | 1.05 kg/kg content in the export product |
|  |  |  | 3 |  | Elastic covered yarn | 1.02 kg/kg content in the export product. |
| Note: The item at Sl.No.2 shall be allowed when the export product is PVC dotted |  |  |  |  |  |  |
| J342 | Lycra Covered with relevant yarn | 1 kg. | 1 |  | Polyester Staple Fibre (1 to 3 DN) | 1.05 kg/kg content in the export product |
|  |  |  | 2 |  | Viscose Staple Fibre (1 to 3 DN) | 1.05 kg/kg content in the export product |
|  |  |  | 3 |  | Lycra/Spandex Yarn | 1.03 kg/kg content in the export product |
|  |  |  | 4 |  | Disperse Dyes 100% Strength | 2% of the wt. of the polyester in the export product |
| Note: If the export product is dyed, then only the import item at Sl.No.4 shall be allowed |  |  |  |  |  |  |
| J343 | Rugs for Horses | 1 kg. | 1 |  | Fabric to be used in the export product | 1.05 kg/kg content in the export product |
| J344 | Girls Skirt | 1 no. | 1 |  | Fabric to be used in the garment | 1.10 sq.mtr. per piece |
| J345 | Ladies Tank Top | 1 pc. | 1 |  | Relevant Knitted fabrics | 1.10 kg/kg content in the export product |
|  |  |  | 2 |  | Relevant Glass Beads/Glass Pipe Beads/Sequins | Net +1% wastage |
| J346 | i) Dyed Pile Fabric ii) Dyed and Printed Pile Fabric/Printed Pile fabric | 1 sq.mtr. 1 Sq.mtr. |  |  | i ) Relevant Pile Grey Fabric | 1.01 Sq.Metre/Sq.Metre content in the export product |
|  |  |  |  |  | ii ) Relevant Pile Dyed Fabric OR Relevant Pile Grey Fabric | 1.01 Sq.Metre / Sq.Metre content in the export product 1.02 Sq.Metre/Sq. Metre content in the export product |
| J347 | Made-ups made from Pile fabrics | 1 Sq.Metre | 1 |  | Relevant Processed Pile Fabrics | 1.01 Sq.Metre/Sq.Metre content in the export product |
| J348 | Polypropylene Spun Yarn | 1 Kg. | 1 |  | Polypropylene staple fibre | 1.03 kg/kg content in the export product |
| J349 | Grey fabrics made from 100% Polyester filament Yarn/Texturised Yarn | 1 kg. | 1 |  | Polyester filament yarn | 1.08 kg/kg content in the export product woven fabric |
|  |  |  |  |  |  | 1.05 kg/kg content in the export product knitted fabric |
| J350 | Dyed twisted mulberry silk yarn (Degummed) | 1 kg. | 1 |  | Mulberry raw silk (gummed and untwisted) | 1.29 kg/kg content in the export product |
| J351 | Children Snow Suit (for age group upto 3 years) | 1 No. | 1 |  | Fabric to be used | 2.00 Sq.Mtr. |
|  |  |  | 2 |  | Fabric as lining | 1.40 sq.mtr. |
| J352 | Viscose Filament Yarn Twisted/Cabled | 1 Kg. | 1 |  | Viscose Filament Yarn | 1.005 kg/kg content in the export product |
| J353 | Blended yarn (containing flax and polyester) | 1 kg | 1 |  | Flax Fibre | 1.66 kg/kg content in the export product |
|  |  |  | 2 |  | Polyster staple fibre | 1.05 kg/kg content in the export product |
| J354 | Deleted |  |  |  |  |  |
| J355 | Cotton Rope made of Cotton yarn Coloured / Dyed | 1 kg | 1 |  | Cotton Yarn Coloured / Dyed | 1.02 kg/kg content in export product |
| J356 | Cotton Duster (Fabric) | 1 kg | 1 |  | Cotton Waste (Motes) excluding the waste extracted at/produced by Carding, Draw frame, Comber, Speed frame and Ring frame stages of conversion of raw cotton from fibre to yarn | 1.90 kg |
| J357 | Cotton Duster (Made-ups) | 1 kg | 1 |  | Cotton Waste (Motes) excluding the waste extracted at/produced by Carding, Draw frame, Comber, Speed frame and Ring frame stages of conversion of raw cotton from fibre to yarn. | 1.94 kg |
| J358 | Dipped Nylon fabrics made out of N6/N66 yarn of 210D-1680D | 1 kg | 1 |  | Grey Nylon Fabric made out of N6/N66 yarn of 210D-1680D OR N6/N66 Multifilament yarn of 210D-1680D | 0.919 kg per kg. Of resultant product 0.951 kg per kg. Of resultant product |
|  |  |  | 2 |  | D. Vinyl pyridine latex (41% DRC) | 0.133 kg per kg of resultant product |
|  |  |  | 3 |  | Resorcinol | 0.0147 kg per kg of resultant product |
|  |  |  | 4 |  | Formaldehyde (37%Dry) | 0.0076 kg per kg of resultant product |
| J359 | Acrylic Coated Fabric (Single Side) | 1 kg | 1 |  | Grey Fabric | 1.05 kg/kg content in the export product |
|  |  |  | 2 |  | Acrylic Emulsions (on solid content basis) | 1.125 kg/kg solid content in the export product |
|  |  |  | 3 |  | Titanium Dioxide | 1.125 kg/kg content in the export product |
|  |  |  | 4 |  | Calcium Carbonate | 1.125 kg/kg content in the export product |
| J360 | Men's Fashion Oversize Half Sleeve Shirt with a minimum length of 31 inches | 1 Pc. | 1 |  | Relevant fabric | 2.6 Sq.mts. |
| J361 | Shawls/ Blankets/ Mufflers made of Pashmina Wool or Pashmina Sliver/ Top/ Roving | 1 kg | 1 |  | Pashmina Woollen Yarn of relevant micron and staple length but below 20 Micron and staple length below 40 mm OR | 1.10 kg |
|  |  |  |  |  | Pashmina Sliver/ Top/ Roving of relevant micron and staple length but below 20 micron and staple length below 40 mm. | 1.20 kg |
| Note : Micron & Staple length should match in the Import & Export. |  |  |  |  |  |  |
| J362 | Acrylic Grey Yarn (Under Worsted System) |  | 1 |  | Acrylic Fibre/Tow | 1.10 kg/kg content in the export product |
| J363 | Men's Suit Set of Two Piece (Jacket + Trouser) with lining | 1 Set | 1 | a) | Fabric Solid OR | 5.25 Sq.mtr. |
|  |  |  | 1 | b) | Fabric Stripe/ Check | 5.72 Sq.mtr. |
|  |  |  | 2 |  | Lining Fabric | 3.15 Sq.mtr. |
|  |  |  | 3 |  | Pocketing Fabric | 1.15 Sq.mtr. |
|  |  |  | 4 |  | Shoulder Pads | Net + 1% |
|  |  |  | 5 |  | Sleeve head OR Felt for Sleeve head & Canvas for Sleeve head | Net + 1% 0.15 Sq.mtr. ------------- 0.1 Sq.mtr. |
|  |  |  | 6 |  | Body felt | 0.25 Sq.mtr. |
|  |  |  | 7 |  | Canvas | 0.65 Sq.mtr. |
|  |  |  | 8 |  | Under Collar felt | 0.10 Sq.mtr. |
|  |  |  | 9 |  | Woven Fusible interlining | 1.00 Sq.mtr. |
|  |  |  | 10 |  | Non-woven fusible inter-lining | 1.00 Sq.mtr. |
|  |  |  | 11 |  | Non-woven non-fusible interlining | 0.10 Sq.mtr. |
|  |  |  | 12 |  | Arm Hole Tape 2-5cm | 2.60 mtr. |
|  |  |  | 13 |  | Brakebind | 0.06 Sq.mtr. |
|  |  |  | 14 |  | Collarbind | 0.04 Sq.mtr. |
|  |  |  | 15 |  | Double fusible net | 0.2 Sq.mtr |
|  |  |  | 16 |  | Label | Net + 1% |
|  |  |  | 17 |  | Button | Net + 1% |
|  |  |  | 18 |  | Thread | 800 Mtr. |
|  |  |  | 19 |  | Tag | Net + 1% |
|  |  |  | 20 |  | Looper | Net + 1% |
|  |  |  | 21 |  | Polybag | Net + 1% |
|  |  |  | 22 |  | Hanger | Net + 1% |
|  |  |  | 23 |  | Hanger Clip | Net + 1% |
|  |  |  | 24 |  | Logo tape upto 2" | 1.25 Mtr. |
|  |  |  | 25 |  | Waist Band Fusible upto 2" | 1.5 Mtr. |
|  |  |  | 26 |  | Loop Fusible upto 1" | 1.00 Mtr. |
|  |  |  | 27 |  | Hook & Eye | Net + 1% |
|  |  |  | 28 |  | Polybag Sticker | Net + 1% |
|  |  |  | 29 |  | Zipper | Net + 1% |
|  |  |  | 30 |  | Slider | Net + 1% |
|  |  |  | 31 |  | Waist Band upto 2" OR Fabric of pocketing description | 1.5 Mtr. ------------- 0.2 Sq.mtr. |
|  |  |  | 32 |  | Ban roll upto 1" width | 1.5 Mtr. |
| J364 | Men's Jacket (Coat) [suit-type] with lining | 1 Piece | 1 | a) | Fabric Solid OR | 3.00 Sq.mtr. |
|  |  |  | 1 | b) | Fabric Stripe/ Check | 3.45 Sq.mtr. |
|  |  |  | 2 |  | Lining Fabric | 2.40 Sq mtr. |
|  |  |  | 3 |  | Pocketing Fabric | 0.50 Sq.mtr |
|  |  |  | 4 |  | Shoulder Pads | Net + 1% |
|  |  |  | 5 |  | Sleeve head OR Felt for Sleeve head & Canvas for Sleeve head | Net + 1% 0.15 Sq.mtr. ------------- 0.1 Sq.mtr. |
|  |  |  | 6 |  | Body felt | 0.25 Sq.mtr. |
|  |  |  | 7 |  | Canvas | 0.5 Sq.mtr. |
|  |  |  | 8 |  | Under Collar felt | 0.10 Sq.mtr. |
|  |  |  | 9 |  | Woven Fusible interlining | 1.0 Sq.mtr. |
|  |  |  | 10 |  | Non-woven fusible inter-lining | 1.00 Sq.mtr. |
|  |  |  | 11 |  | Non-woven non-fusible interlining | 0.10 Sq.mtr. |
|  |  |  | 12 |  | Arm Hole Tape 2-5cm | 2.60 mtr. |
|  |  |  | 13 |  | Brakebind | 0.03 Sq.mtr |
|  |  |  | 14 |  | Collarbind | 0.04 Sq.mtr. |
|  |  |  | 15 |  | Double fusible net | 0.2 Sq.mtr |
|  |  |  | 16 |  | Label | Net +1% |
|  |  |  | 17 |  | Button | Net +1% |
|  |  |  | 18 |  | Thread | 400 mtr. |
|  |  |  | 19 |  | Tag | Net +1% |
|  |  |  | 20 |  | Looper | Net +1% |
|  |  |  | 21 |  | Polybag | Net +1% |
|  |  |  | 22 |  | Hanger | Net +1% |
|  |  |  | 23 |  | Hanger Clip | Net +1% |
|  |  |  | 24 |  | Polybag Sticker | Net +1% |
| J365 | Men's Trouser (Suit-Type) with lining | 1 Piece | 1 | a) | Fabric Solid OR | 2.25 Sq.mtr. |
|  |  |  | 1 | b) | Fabric Stripe/ Check | 2.6 Sq.mtr. |
|  |  |  | 2 |  | Lining Fabric | 0.75 Mtr. |
|  |  |  | 3 |  | Pocketing Fabric | 0.65 Sq.mtr. |
|  |  |  | 4 |  | Non-woven fusible inter-lining | 0.10 Sq.mtr. |
|  |  |  | 5 |  | Brakebind OR Fabric of Pocketing description | 0.03 Sq.mtr ------------- 0.2 Sq.mtr |
|  |  |  | 6 |  | Lebel | Net +1% |
|  |  |  | 7 |  | Button | Net +1% |
|  |  |  | 8 |  | Thread | 400 mtr. |
|  |  |  | 9 |  | Tag | Net +1% |
|  |  |  | 10 |  | Looper | Net +1% |
|  |  |  | 11 |  | Polybag | Net +1% |
|  |  |  | 12 |  | Hanger | Net +1% |
|  |  |  | 13 |  | Hanger Clip | Net +1% |
|  |  |  | 14 |  | Logo Tape upto 2" | 1.50 Mtr. |
|  |  |  | 15 |  | Canvas | 0.15 Sq.mtr. |
|  |  |  | 16 |  | Waist band fusible | 1.58 Mtr. |
|  |  |  | 17 |  | Loop Fusible upto 1" | 1.0 Mtr. |
|  |  |  | 18 |  | Hook & eye | Net +1% |
|  |  |  | 19 |  | Polybag Sticker | Net +1% |
|  |  |  | 20 |  | Zipper | Net +1% |
|  |  |  | 21 |  | Slider | Net +1% |
|  |  |  | 22 |  | Waist band upto 2" | 1.5 Mtr. |
|  |  |  | 23 |  | Ban roll upto 1" width | 1.5 Mtr. |
| J366 | Boy's Suit Set (Coat + Pant) with lining | 1 Set | 1 | a) | Fabric Solid OR | 4.2 Sq.mtr. |
|  |  |  | 1 | b) | Fabric Stripe/ Check | 4.72 Sq.mtr. |
|  |  |  | 2 |  | Lining Fabric | 2.40 Sq.mtr. |
|  |  |  | 3 |  | Pocketing Fabric | 0.9 Sq.mtr. |
|  |  |  | 4 |  | Shoulder Pads | Net +1% |
|  |  |  | 5 |  | Sleeve head OR Felt for Sleeve head & Canvas for Sleeve head | Net +1% 0.15 Sq.mtr. ------------- 0.1 Sq.mtr. |
|  |  |  | 6 |  | Body felt | 0.2 Sq.mtr. |
|  |  |  | 7 |  | Canvas | 0.50 Sq.mtr. |
|  |  |  | 8 |  | Under Collar felt | 0.10 Sq.mtr. |
|  |  |  | 9 |  | Woven Fusible interlining | 0.55 Sq.mtr. |
|  |  |  | 10 |  | Non-woven fusible inter-lining | 1.00 Sq.mtr. |
|  |  |  | 11 |  | Non-woven non-fusible interlining | 0.10 Sq.mtr. |
|  |  |  | 12 |  | Arm Hole Tape 2-5cm | 2.4 mtr. |
|  |  |  | 13 |  | Brakebind | 0.06 Sq.mtr. |
|  |  |  | 14 |  | Collarbind | 0.04 Sq.mtr. |
|  |  |  | 15 |  | Double fusible net | 0.2 Sq.mtr. |
|  |  |  | 16 |  | Label | Net +1% |
|  |  |  | 17 |  | Button | Net +1% |
|  |  |  | 18 |  | Thread | 600 Mtr. |
|  |  |  | 19 |  | Tag | Net +1% |
|  |  |  | 20 |  | Looper | Net +1% |
|  |  |  | 21 |  | Polybag | Net +1% |
|  |  |  | 22 |  | Hanger | Net +1% |
|  |  |  | 23 |  | Hanger Clip | Net +1% |
|  |  |  | 24 |  | Logo tape upto 2" | 1.25 Mtr. |
|  |  |  | 25 |  | Waist Band fusible upto 2" | 1.4 Mtr. |
|  |  |  | 26 |  | Loop Fusible upto 1" | 1.37 Mtr. |
|  |  |  | 27 |  | Hook & Eye | Net +1% |
|  |  |  | 28 |  | Polybag Sticker | Net +1% |
|  |  |  | 29 |  | Zipper | Net +1% |
|  |  |  | 30 |  | Slider | Net +1% |
|  |  |  | 31 |  | Waist Band upto 2" OR Fabric of pocketing description | 1.4 Mtr. ------------ 0.2 Sq.mtr. |
|  |  |  | 32 |  | Ban roll upto 1" width | 1.4 Mtr. |
| J367 | Boy's Jacket (Coat) (Suit-type) with lining | 1 Piece | 1 | a) | Fabric Solid OR | 2.4 Sq.mtr. |
|  |  |  | 1 | b) | Fabric Stripe/ Check | 2.75 Sq.mtr. |
|  |  |  | 2 |  | Lining Fabric | 2.00 Sq.mtr. |
|  |  |  | 3 |  | Pocketing Fabric | 0.4 Sq.mtr. |
|  |  |  | 4 |  | Shoulder Pads | Net +1% |
|  |  |  | 5 |  | Sleeve head OR Felt for Sleeve head & Canvas for Sleeve head | Net +1% 0.12 Sq.mtr. ------------ 0.1 Sq.mtr. |
|  |  |  | 6 |  | Body felt | 0.25 Sq.mtr. |
|  |  |  | 7 |  | Canvas | 0.4 Sq.mtr. |
|  |  |  | 8 |  | Under Collar felt | 0.10 Sq.mtr. |
|  |  |  | 9 |  | Woven Fusible interlining | 0.8 Sq.mtr. |
|  |  |  | 10 |  | Non-woven fusible inter-lining | 0.8 Sq.mtr. |
|  |  |  | 11 |  | Non-woven non-fusible interlining | 0.10 Sq.mtr. |
|  |  |  | 12 |  | Arm Hole Tape 2-5cm | 2.5 mtr. |
|  |  |  | 13 |  | Brakebind | 0.03 Sq.mtr. |
|  |  |  | 14 |  | Collarbind | 0.04 Sq.mtr. |
|  |  |  | 15 |  | Double fusible net | 0.2 Sq.mtr. |
|  |  |  | 16 |  | Label | Net +1% |
|  |  |  | 17 |  | Button | Net +1% |
|  |  |  | 18 |  | Thread | 300 Mtr. |
|  |  |  | 19 |  | Tag | Net +1% |
|  |  |  | 20 |  | Looper | Net +1% |
|  |  |  | 21 |  | Polybag | Net +1% |
|  |  |  | 22 |  | Hanger | Net +1% |
|  |  |  | 23 |  | Hanger Clip | Net +1% |
|  |  |  | 24 |  | Polybag Sticker | Net +1% |
|  |  |  | 25 |  | Zipper | Net +1% |
| J368 | Boy's Trouser (Suit-type) with lining | 1 Piece | 1 | a) | Fabric Solid OR | 1.8 Sq.mtr. |
|  |  |  | 1 | b) | Fabric Stripe/ Check | 2.05 Sq.mtr. |
|  |  |  | 2 |  | Lining Fabric | 0.65 Mtr. |
|  |  |  | 3 |  | Pocketing Fabric | 0.6 Sq.mtr. |
|  |  |  | 4 |  | Non-woven fusible inter-lining | 0.10 Sq.mtr. |
|  |  |  | 5 |  | Brakebind | 0.03 Sq.mtr. |
|  |  |  | 6 |  | Lebel | Net +1% |
|  |  |  | 7 |  | Button | Net +1% |
|  |  |  | 8 |  | Thread | 300 Mtr. |
|  |  |  | 9 |  | Tag | Net +1% |
|  |  |  | 10 |  | Looper | Net +1% |
|  |  |  | 11 |  | Polybag | Net +1% |
|  |  |  | 12 |  | Hanger | Net +1% |
|  |  |  | 13 |  | Hanger Clip | Net +1% |
|  |  |  | 14 |  | Logo Tape upto 2" | 1.4 Mtr. |
|  |  |  | 15 |  | Canvas | 0.15 Sq.mtr. |
|  |  |  | 16 |  | Waist band fusible upto 2" | 1.4 Mtr. |
|  |  |  | 17 |  | Loop Fusible upto 1" | 1.0 Mtr. |
|  |  |  | 18 |  | Hook & eye | Net +1% |
|  |  |  | 19 |  | Polybag Sticker | Net +1% |
|  |  |  | 20 |  | Zipper | Net +1% |
|  |  |  | 21 |  | Slider | Net +1% |
|  |  |  | 22 |  | Waist band upto 2" OR Fabric of Pocketing Description | 1.4 Mtr. ------------ 0.2 Sq.mtr. |
|  |  |  | 23 |  | Ban roll upto 1" width | 1.4 Mtr. |
| J369 | Polyester popcorn/Condux | 1 kg | 1 |  | Polyester Waste | 1.02 kg |
| J370 | Nonwoven fabrics made of manmade fibre | 1 kg | 1 |  | Relevant Man-made fibre | 1.17 kg/kg content in the export product |
| J371 | Processed silk (Silk Sliver) | 1 kg | 1 |  | Silk Waste from cocoons and reelers in gummed form | 2.22 kg |
| J372 | 100% Flax/Linen yarn (Auto cone spliced) | 1 kg | 1 |  | 100% Flax/Linen yarn (Uneven & Knotted) | 1.045 Kg/Kg. of export item |
| J373 | Raw cotton (not carded or combed) | 1 kg | 1 |  | Grey cotton fabrics (75-95 GSM)- suitable for packing cotton bales | 0.0035 kgs |
|  |  |  | 2 |  | PP/HDPE/LDPE/LLDPE granules, suitable for making plastic woven fabrics for packing cotton bales | 0.00229 kgs |
|  |  |  | 3 |  | Mild Steel Coils (Width 1/2 inch - 3/4 inch) | 0.00955 kgs |
|  |  |  | 4 |  | PVC Straps | 1.05 kgs/kg content of import item in the export product |
| Note: (1) It may be noted that a cotton bale would be packed either in grey cotton fabrics or in plastic woven fabrics, but never in both. Hence, import quantity at Sr. No. 1 and Sr. No.2 to be computed taking into accout the quantity of export product backed in plastic woven fabrics (for import item No.2). Note: (2) It may be noted that a cotton bale would be strapped either in Steel Coil or in PVC Strap, but not in both. Hence, import quantity at Sr. No.3 and/Or Sr. No. 4 to be computed taking into account the quantity of export product strapped using Mild Steel Coil (Width 1/2 inch-3/4 inch) and/or quantity of export product in PVC Straps (for import item No.4). |  |  |  |  |  |  |